Disaster Experience, Governance, and Housing Robustness:

A Two-Stage Framework Across 150 Countries

Yasuhiro Kawasoe

The World Bank

Abstract

Building quality is widely recognized as the primary determinant of disaster mortality and asset loss, yet what drives structural safety across countries remains poorly understood. Countries at similar income levels show housing resilience rates that differ by 50 to 70 percentage points, a divergence that income alone cannot explain.

This paper addresses these questions through a novel dataset covering residential building robustness in 150 countries, linked to indicators of poverty, national income, institutional quality, construction-sector regulation, and cumulative hazard exposure from 2000 to 2020. A two-stage housing resilience ladder framework is proposed and tested, arguing that the binding constraints on housing improvement shift fundamentally across development stages. In Stage 1, eliminating fragile housing, household poverty is the dominant constraint, with strong nonlinear effects. In Stage 2,  achieving robust, engineered construction, poverty becomes statistically insignificant, while construction-sector regulatory quality emerges as the critical determinants.

The cross-national evidence reveals that associations between disaster exposure and housing robustness are hazard-specific and moderated by governance quality. Countries with greater cumulative earthquake exposure show systematically higher shares of non-fragile and robust housing, particularly where governance quality is higher. Storm and flood exposure shows weaker direct effects but is associated with a "reconstruction leap" in high-poverty contexts, a disproportionate reduction in the lowest-quality housing stock, consistent with humanitarian reconstruction pressure. This divergence suggests that hazard type shapes the governance mechanisms through which resilience improvements may occur. These findings carry direct implications for how DRR investments should be sequenced, targeted, and governed across development stages.

Keywords: Housing Resilience, Disaster Resilience, Building Codes, Disaster Learning, Poverty Constraints, Institutional Quality, Construction Governance

# Introduction

## The Housing Vulnerability Challenge

The pronounced disparity in impacts among recent global earthquakes underscores a fundamental reality: the quality of the built environment, particularly housing, is the primary determinant of disaster damage and loss of life. On January 1, 2024, the magnitude 7.6 Noto Peninsula earthquake in Japan resulted in 228 direct fatalities and estimated economic losses ranging from $7.3 billion to $17.3 billion, with 6,536 buildings destroyed [1]. While the Noto earthquake caused substantial damage, its human toll and physical destruction remain relatively low when contrasted with the February 6, 2023, twin earthquakes (magnitudes 7.8 and 7.5) that struck Türkiye and the Syrian Arab Republic. These catastrophic events caused more than 56,000 deaths, damaged at least 230,000 buildings, and incurred approximately $40 billion in direct losses [2] [3]. Although geographic exposure and seismic intensity varied, the stark difference in human impact is largely due to the disparity in building quality, which is influenced by building code systems, enforcement capacity, and construction-sector governance that differ fundamentally between the two countries [4]. Understanding why such divergence persists across countries is a core, unresolved question in disaster risk science and is directly relevant to the Sendai Framework's call for a systematic understanding of risk [5].

Globally, the scale of housing inadequacy presents a critical development challenge. More than 2.8 billion people live in inadequate housing, including 1.1 billion people in informal settlements and slums with the majority located in developing countries [6]. Many of these settlements are located in high-hazard zones, such as coastal floodplains, seismic belts, and cyclone corridors [7]. This makes the gap between housing quality and structural safety a central disaster risk reduction (DRR) governance challenge. This physical vulnerability compounds catastrophic financial risks: annual disaster losses exceed $2.3 trillion including indirect and ecosystem impacts [8]. For low-income and poor households, housing frequently represents a substantial portion of their total wealth, often making its destruction in a disaster a devastating and inescapable poverty shock [9] [10].

The central policy challenge in addressing this global housing vulnerability is that affordability and disaster-resilience are often framed as competing, rather than complementary, objectives [11] [12]. Affordable housing programs in developing countries often prioritize rapid and inexpensive construction to meet acute housing shortages, frequently at the expense of adequate regulation or oversight to ensure resilience against natural hazard [13]. While the disaster risk management community advocates for disaster-resilient construction, compliance with structural safety standards remains low across much of the Global South, where building codes are adopted on paper but enforced selectively or not at all [4] [14] For instance, the implementation of earthquake-resistant construction can result in an increase in building costs by a minimum of 10%, primarily due to the necessity of utilizing stronger materials, adopting advanced technologies, and adhering to more stringent seismic codes [15]. This premium is seldom absorbed voluntarily without the presence of conducive regulatory and financial conditions.

Yet framing this as a strict trade-off may be misleading and overly simplistic. Poor households construct dwellings with fragile materials (mud mortar, unreinforced masonry) not because they prefer risk but because they face binding liquidity constraints and limited access to financing [16]. In this view, the adoption of building codes is contingent upon the presence of economic enabling conditions, irrespective of enforcement intentions. However, it is imperative to ascertain whether poverty is indeed the sole and overarching impediment to structural safety investment. Alternatively, do other factors, such as inadequate construction-sector regulation, limited disaster risk awareness, and limited local construction capacity, matter independently and significantly, and do their relative weights shift as countries develop? A critical question to address is whether the same constraint that hinders the elimination of fragile housing also impedes the transition to fully engineered, robust construction. The present paper proposes and tests a two-stage housing resilience framework to address these questions systematically.

## The Puzzle: Heterogeneous Resilience at Similar Income Levels

Despite shared economic constraints, housing resilience varies dramatically across countries at similar income levels. Viet Nam (GDP per capita $14,415 PPP) has only 30.7% of its housing stock meeting moderate resilience standards, compared to 86% in Indonesia ($14,470 PPP) — an almost identical income level.  Sri Lanka ($13,752 PPP) reaches 17.3%, against Guatemala's ($12,640 PPP) 81.4%. Disparities of 50–70 percentage points among income-comparable countries defy simple income-based explanations. Prior studies have examined income, governance, and disaster experience as separate determinants of housing vulnerability, but none has systematically tested their joint and stage-specific effects across the full income spectrum using comparable structural building data. This paper addresses that gap.

### Disaster exposure and learning

The first mechanism holds that repeated disaster experience can catalyze systemic change in construction practice, building regulation, and institutional capacity for structural safety. Countries with limited prior hazard exposure may lack the political incentive to invest proactively in code development and enforcement [17]. The historical record contains compelling examples. The 1960 Valdivia Earthquake (Mw 9.5) catalyzed Chile's first national seismic standard (1972), continuously revised after subsequent events. The 2010 Maule earthquake (Mw 8.8) resulted in approx. 500 deaths despite affecting 80% of the population, with modern reinforced concrete showing limited collapse rates [18].  Similarly, the 1995 Great Hanshin-Awaji Earthquake in Japan informed the promotion of seismic retrofitting through the enactment of specific legislation [19]. These cases suggest that earthquake experience, when mediated by sufficient institutional capacity, can produce durable improvements in building-level structural safety — a mechanism with direct empirical relevance to Sendai Priority 4's Build Back Better concept [5].

However, many disasters fail to trigger improvements. Nepal's 2015 Gorkha Earthquake (Mw 7.8, 9,000+ deaths) led to the establishment of a National Reconstruction Authority with explicit "Build Back Better" mandates and code-compliant reconstruction grants. Yet implementation faced challenges: expert engineers were unavailable in rural areas, homeowners reverted to traditional methods, and 15.2% of reconstructed buildings were non-compliant [20]. The Philippines show mixed patterns: 2009 floods catalyzed policy reform (RA 10121) [21], but 2013 Typhoon Haiyan reconstruction showed 78% poor quality, with pre-disaster vulnerability predicting post-disaster outcomes [22]. These failures point to a governance gap between disaster experience and structural improvement, and raise the central question this paper addresses empirically: does disaster learning generalize uniformly across income levels and hazard types, or is it fundamentally mediated by institutional capacity and economic enabling conditions?

### Institutional capacity and construction governance

The second mechanism is that effective governance is crucial to enable code enforcement, secure land-use regulation, and control corruption [23]. A key issue is that developing countries often adopt modern building codes but fail to enforce them effectively due to limited inspection capacity and pervasive informal construction [4]. Empirical evidence suggests that corruption acts as a significant moderating factor that undermines the effectiveness of policies [24]. A large panel study (1980–2020) demonstrates that higher levels of corruption significantly increase the number of disaster-related fatalities as a share of the population. This devastating impact is attributed to corruption weakening the institutional underpinnings that shape the quality of public infrastructure and governing the adherence to vital safety measures like building codes. The study further finds this effect is significantly stronger in developing countries. [25]. Addressing this enforcement gap is explicitly recognized under Sendai Framework Priority 3, which calls for investment in DRR that strengthens the structural and institutional conditions for resilience [5].

This evidence implies that the success of post-disaster reforms (i.e., achieving "Build Back Better") is fundamentally dependent on concurrent efforts to strengthen governance and integrity, as corruption translates hazard exposure directly into increased human mortality. A critical question remains: do broad governance indicators, such as general government effectiveness, determine housing robustness, or is it the quality of construction-specific institutions, such as building code stringency, inspector training, and contractor certification?

### Economic constraints

Beyond household poverty, economic structures create binding constraints through three channels. First, underdeveloped financial markets limit households' capacity to invest in quality materials: in the absence of accessible mortgages, households engage in incremental building that precludes integrated resilient design [26]  [27]. Retrofitting financing is particularly scarce  [28]. Second, large informal construction sectors, characteristic of lower-income economies, mean that laborers often lack formal training in resilient techniques, generating poor outcomes even when materials are adequate. Third, insecure land tenure discourages long-term investment in durable housing and prevents households from accessing property-collateralized credit, perpetuating vulnerability  [29]. These mechanisms are not mutually exclusive, and their relative importance may shift across development stages, a possibility no existing study has tested systematically across the full income spectrum.

## Research Questions

This paper addresses four interrelated empirical questions aimed at identifying the mechanisms associated with housing resilience.

- Does the binding constraint on structural safety shift across development stages?

- Is disaster exposure associated with housing improvements, and does this vary by hazard type?

- Is the association between disaster exposure and housing quality mediated by governance and income?

- What are the independent contributions of general governance versus construction-sector-specific institutions?

# Data and Measurement

This study assembles a cross-national dataset for 150 economies, linking a novel measure of structural housing robustness to national indicators of economic development, poverty, institutional quality, construction-sector regulation, and cumulative hazard exposure. No prior cross-national study has combined building-stock taxonomy data with this full set of DRR-relevant governance and economic indicators at this scale. Data sources, variable construction, and measurement decisions are described below; economies excluded due to missing variables are listed in Appendix Table A-10.

## Dependent Variable: Housing Robustness

### Construction from GEM Building Taxonomy

Housing robustness is measured using the Global Exposure Model (GEM) developed by the Global Earthquake Model Foundation [30] [31], which provides comprehensive building stock data for 215 countries. The GEM database covers residential building taxonomy (structural material, lateral load-resisting system, height band, and ductility design level), unit counts, and replacement costs at the national level. The 2023 release, based on 2021 economic values and updated building inventories, is used.

The assignment of GEM taxonomy strings to vulnerability tiers (fragile, medium, and robust) follows the engineering classification developed by Middelanis et al. [32], which maps structural characteristics to expected performance under earthquake, flood/storm surge, and wind hazards separately, then aggregates these into a combined tier. Two binary outcome measures are constructed from this classification: a non-fragile indicator (robust or medium quality, capturing Stage 1 of the housing resilience ladder, or elimination of fragile housing) and a robust-only indicator (capturing Stage 2, or achievement of engineered construction standards). This two-threshold approach directly operationalizes the two-stage framework and allows the binding constraints on each transition to be estimated independently.

The fragile-to-robust ordering was verified across all 123 taxonomy entries in the GEM dataset; Appendix Table A-9 presents a representative selection of 23 entries. Unreinforced and earthen masonry construction types are confirmed fragile, producing the highest collapse rates in documented post-earthquake reconnaissance including Bam 2003, the 2015 Gorkha earthquake in Nepal, and the 2023 Kahramanmaraş earthquakes in Turkey [33] [34] [35]. Confined and reinforced masonry types are confirmed median, consistent with experimental evidence on confinement-derived ductility improvements [36] [37] [38]. Ductile reinforced concrete systems, including dual frame-wall systems (LDUAL) and moment frames (LFM), and steel frames are confirmed robust. RC infilled frames (LFINF) are classified as medium rather than robust, reflecting the well-documented vulnerability of masonry infill panels to short-column shear failures and soft-story mechanisms that prevent the full development of frame ductility under seismic loading. In the combined hazard tier, where earthquake, flood/storm, and wind performance diverge, the worst single-hazard outcome generally governs, a conservative aggregation consistent with the multi-hazard exposure of the global building stock.

Buildings are weighted by replacement cost rather than unit count, following the approach of Middelanis et al. [32], as replacement cost captures economic exposure, the primary metric for disaster risk quantification in the DRR literature. A high-value engineered apartment building represents substantially greater economic exposure than multiple low-value informal dwellings. GDP per capita is controlled in regressions to ensure cost-weighting does not mechanically capture national wealth effects. It is acknowledged that replacement-cost weighting may understate the structural vulnerability of the poorest households, who disproportionately occupy fragile housing with low replacement value; unit-weighted results presented in Appendix Table A-5 confirm that the main findings hold but reveal a larger residual fragile housing share among lower-value dwelling units.

### Measurement Limitations and Validity

Two measurement limitations are acknowledged. First, low-rise RC buildings (one to three stories) receive a median combined tier despite robust seismic performance, because ground-floor flood exposure governs the combined classification under the conservative worst-case aggregation logic. This classification is conservative with respect to the seismic results: to the extent that low-rise RC is over-penalized in the combined tier, the estimated effect of seismic learning on housing robustness is likely understated rather than overstated. Second, and more fundamentally, the taxonomy classifies structural typology rather than construction quality within a type: a ductile RC frame built with substandard materials or inadequate supervision may perform below its median tier, while a well-executed confined masonry structure may exceed its medium assignment. This within-typology quality variance is the irreducible limitation of any globally consistent building stock dataset at this scale, and one that cannot be resolved without systematic engineering surveys across all 150 countries.

Despite these limitations, the GEM taxonomy represents the most comprehensive global dataset on housing structural characteristics currently available. Alternative data sources have more severe limitations. Household surveys (Demographic and Health Survey [DHS], Living Standards Measurement Study [LSMS]), national censuses) typically collect only roof, wall, and floor materials without information on structural systems, height, or lateral load resistance. Remote sensing (satellite imagery, street view) can identify building footprints, height, and roof materials but cannot observe structural systems, reinforcement details, or foundation types. Engineering surveys provide ground-truth data on construction quality but exist only for selected cities or post-disaster assessments, lacking the systematic global coverage necessary for cross-national analysis at this scale.

### Alternative Dependent Variable (Robustness Check)

The main dependent variable derives from GEM building taxonomy classifications, which rely on expert assessments of structural systems, materials from different sources. While comprehensive in coverage and technically rigorous, the measure could potentially reflect taxonomy categorization choices rather than actual housing conditions. As a partial check on whether findings are sensitive to the GEM measurement approach, the core model is re-estimated using the housing deprivation component of the UN's Multidimensional Poverty Index (MPI). MPI housing deprivation measures the percentage of population living in dwellings with inadequate flooring, roofing, or walls, derived from household survey data in 96 countries. This represents direct observation of housing materials through household interviews rather than expert inference from building taxonomy. The author constructs the dependent variable as 100 - MPI Housing Deprivation (higher values = better housing quality) and re-estimate the baseline specification.

## Independent Variables

Unless otherwise noted, all independent variables use the most recent observation available prior to 2023 for each country, minimizing temporal mismatch with the GEM building stock data while avoiding endogeneity from post-measurement policy changes.

Economic Development: GDP per capita, PPP-adjusted (constant 2021 USD) from World Bank World Development Indicators (WDI). The log transformation is applied to account for the diminishing marginal relationship between income and housing quality and to reduce the influence of outliers among high-income economies.

Poverty: Poverty Gap at $4.20/day (2021 PPP), from the World Bank Poverty and Inequality Platform. The poverty gap rather than the headcount ratio is used to capture the depth of deprivation and to reduce multicollinearity with GDP per capita. The $4.20/day threshold corresponds to the median national poverty line among lower-middle-income countries [39], providing a broader measure of binding economic constraint than the extreme poverty line($3.00/day). Robustness checks using the headcount ratio and alternative thresholds are reported in Appendix Table A-2.

Urbanization: Proportion of population living in urban areas, from World Bank World Development Indicators. Urbanization is included to control for the compositional effect of urban areas having systematically higher shares of engineered construction, denser code enforcement infrastructure, and greater household access to formal building materials, all of which independently predict housing robustness beyond income and governance indicators.

Institutional Quality: Government Effectiveness from the Worldwide Governance Indicators (WGI), year 2024 update. Government Effectiveness captures "perceptions of the quality of public services, the quality of the civil service and the degree of its independence from political pressures, the quality of policy formulation and implementation, and the credibility of the government's commitment to such policies" [40]. The paper selects Government Effectiveness as it most directly captures the state's capacity to formulate, implement, and enforce the complex technical regulations, land-use policies, and public service provisions that underpin a resilient housing stock.

Building Quality Control: Building Quality Control Index (BQCI) from the World Bank Doing Business Archive (2020 vintage, last year published prior to indicator discontinuation in 2021). The index measures construction-sector regulatory quality on a 0–15 scale across six dimensions: quality of building regulations; pre-construction quality control; quality control during construction; post-construction inspection; liability and insurance regimes; and professional certification requirements. Despite discontinuation, the BQCI remains the most comprehensive cross-national measure of construction-sector-specific governance available, and its 2020 vintage is temporally aligned with the GEM building stock data. No equivalent indicator at comparable country coverage exists in the post-2021 period.

Disaster Experience: Disaster experience is operationalized using EM-DAT, cumulative event counts for 2000–2020, aligned with the GEM data vintage [41]. EM-DAT records events satisfying at least one of the following criteria: ten or more deaths; one hundred or more people affected; declaration of a state of emergency; or a call for international assistance. This threshold captures events plausibly large enough to trigger policy responses and institutional learning while excluding localized incidents unlikely to generate systemic change. Three hazard types directly affecting housing structural integrity are examined: earthquakes, storms (tropical cyclones, hurricanes, and typhoons), and floods. The primary exposure metric is the cumulative count of qualifying events per hazard type over 2000–2020, log-transformed in order to account for the highly skewed distribution of event frequency across countries. To verify that results are not sensitive to this operationalization, robustness checks employ binary disaster occurrence indicators, normalized event counts per unit area, and cumulative death tolls (Appendix Table A-4).

## Summary Statistics

Table 1 presents summary statistics for the sample of 150 countries. Housing robustness varies substantially across countries: the non-fragile share ranges from 2% to 100% by replacement cost, and only 28% of global housing stock meets the robust standard, meaning that nearly three-quarters of the world's residential building value remains exposed to significant structural failure risk under disaster conditions. This distribution provides substantial cross-national variation for identifying the determinants of each transition stage. Disaster exposure is highly uneven, with storm event counts ranging from 0 to 294 over 2000–2020, justifying log transformation. Socioeconomic variables span similarly wide ranges: poverty gap from 0% in advanced economies to 60.3% (Democratic Republic of Congo, 2020); GDP per capita from $1,036 to $130,049 PPP; ensuring that both Stage 1 and Stage 2 transitions are empirically observable within the sample.

Table 1 Summary Statistics

| Variable | N | Mean | Std. Dev. | Min | Max |
| --- | --- | --- | --- | --- | --- |
| A. Housing Robustness Measures (Cost-Weighted Proportion) |  |  |  |  |  |
| Non-Fragile | 150 | 0.52 | 0.28 | 0.02 | 1.00 |
| Robust Only | 150 | 0.28 | 0.243 | 0.00 | 1.00 |
| B. Socioeconomic and Governance Variables |  |  |  |  |  |
| Poverty Gap (%, $4.20/day, 2021 PPP) | 150 | 8.58 | 13.05 | 0.00 | 60.30 |
| GDP per Capita (2021, International USD, PPP) | 150 | 23,733 | 24,884 | 1036 | 130,049 |
| Urban Population (%) | 150 | 59.34 | 21.19 | 16.98 | 99.28 |
| Government Effectiveness (WGI, 2024) | 150 | -0.06 | 0.94 | -2.23 | 2.13 |
| Building Quality Control Index (0-15, Doing Business 2020) | 150 | 10.28 | 3.11 | 0.00 | 15.00 |
| C. Disaster Experience (2000-2020, Event Counts) |  |  |  |  |  |
| Earthquake Events | 150 | 3.23 | 10.53 | 0.00 | 97 |
| Storm Events | 150 | 12.17 | 33.31 | 0.00 | 294 |
| Flood Events | 150 | 19.91 | 28.90 | 0.00 | 201 |

Notes: All variables represent the most recent available year before 2023 for each country. Housing robustness measures reflect GEM taxonomy data (2021-2022 collection). Disaster counts cover 2000-2020. Poverty and GDP data are primarily from 2018-2023.

## Preliminary Bivariate Relationships

Figure 1 presents bivariate scatterplots; the full correlation matrix is in Appendix Table A-1. Non-fragile and robust housing shares are positively correlated with log GDP per capita and government effectiveness, and negatively correlated with the poverty gap, patterns consistent with theoretical expectations. The BQCI shows a stronger correlation with the robust-only share than with the non-fragile share, foreshadowing the Stage 2 governance finding. Earthquake exposure is uncorrelated with GDP per capita, consistent with the largely exogenous geographic determination of seismic exposure. Flood exposure is negatively correlated with robust housing, GDP, and governance effectiveness, suggesting that flood-prone countries face compounding structural vulnerabilities. These differential hazard patterns motivate the disaggregated disaster learning specifications in Section 3.

Figure 1  Key Bivariate Relationships

Note: Scatterplots showing Non-fragile housing vs. (A) Poverty Gap; (B)  log GDP per capita; and  (C) Government Effectiveness. Each point represents one country (N=150). Fitted lines show OLS relationships without controls.

# Empirical Strategy

This paper employs a nested cross-sectional estimation strategy to identify the structural, economic, and governance determinants of housing resilience across 150 countries. The analysis proceeds in three sequential phases: baseline models establishing the independent contributions of poverty, income, urbanization, and governance to each stage of the housing resilience ladder (Section 3.1); disaster learning models testing whether cumulative hazard exposure drives measurable improvements in structural safety (Section 3.2); and interaction models examining whether disaster learning is mediated by poverty levels and governance quality (Section 3.3). All specifications are estimated in parallel for two dependent variables: the non-fragile housing share and the robust-only housing share, enabling direct comparison of binding constraints across the two development stages, as illustrated in Figure 2. Given the cross-sectional design, estimated coefficients should be interpreted as conditional correlations rather than causal effects; the direction of causality is grounded in the theoretical prior that governance institutions and income levels predate and shape the accumulated housing stock, rather than the reverse.

Figure 2 The Two-Stage Housing Resilience Ladder Framework

## Phase 1: Structural Baseline Models

The baseline specification tests the independent contribution of poverty, income, urbanization, and governance to structural housing safety, using a nested modeling strategy that adds explanatory variables sequentially. This approach serves two purposes: it reveals how coefficient estimates evolve as additional controls are introduced, providing evidence on whether poverty and governance effects are robust or confounded. It directly tests the two-stage hypothesis by comparing which variables enter as significant predictors of Stage 1 (non-fragile housing) versus Stage 2 (robust housing). The baseline specification relates housing robustness to economic and institutional determinants:

| Vi=α+β1POVi+β2GDPi++β3URB+β4GOVi+β5BQCIi+r=16δrREGIONir+ϵi | ( SEQ Equation \* ARABIC 1) |
| --- | --- |

where Vi represents housing robustness in country i (cost-weighted proportion of non-fragile or robust housing stock), POVi is the Poverty Gap at $4.20/day (2021 PPP), GDPi is log GDP per capita (2021 PPP),  URBi is the proportion of Urban Population, GOVi captures government effectiveness from the World Governance Indicators, and BQCIi is the Building Quality Control Index measuring construction-specific regulatory capacity. All continuous variables are mean-centered prior to estimation, which facilitates interpretation of main effects in the interaction models. REGIONir​ are regional fixed effects indicators for seven World Bank geographical classifications (Europe & Central Asia, Middle East & North Africa, Sub-Saharan Africa, Latin America & Caribbean, North America, South Asia, and East Asia & Pacific as the reference category), The error term ϵi captures unobserved determinants of housing quality.

All models are estimated using ordinary least squares (OLS) with heteroskedasticity-robust standard errors, which correct for non-constant error variance commonly arising in cross-national datasets where variation in housing outcomes differs systematically across development levels. While cross-sectional OLS cannot establish causality, it provides consistent estimates of conditional correlations under the assumption that the control set captures the primary confounding pathways. This assumption is assessed through multiple robustness checks detailed in Section 4.5.

The paper uses the sixed nested specification sequence to observe how coefficient estimates evolve as the model accounts for additional factors. Model 1 includes only the poverty gap and regional fixed effects, establishing whether poverty alone predicts housing robustness. Model 2 adds a quadratic poverty term to test for nonlinear threshold effects, specifically whether poverty's negative association with housing quality diminishes at extreme deprivation levels, consistent with the Stage 1 hypothesis. Model 3 introduces log GDP per capita to disentangle the independent contributions of national wealth and household poverty, which are highly correlated but theoretically distinct. Model 4 adds urban population share to test whether urbanization independently predicts structural safety beyond income. Model 5 introduces Government Effectiveness to assess broad institutional capacity. Model 6 adds BQCI alongside Government Effectiveness, enabling identification of whether construction-sector-specific regulatory capacity matters independently of general governance quality.

## Phase 2: Disaster Learning Models

To test whether cumulative disaster exposure is associated with measurable improvements in structural safety, the empirical precondition for effective Build Back Better policies under Sendai Framework Priority 4 [5], two complementary specifications are estimated. Model A disaggregates exposure by hazard type to identify whether seismic, storm, and flood events generate distinct learning pathways; Model B uses an aggregate exposure measure to test whether the overall frequency of disasters, regardless of type, is sufficient to catalyze structural improvement. Testing both specifications is analytically necessary: if hazard-specific coefficients in Model A diverge significantly, the aggregate measure in Model B would mask this heterogeneity and overstate the generality of disaster learning.

### Model A: Disaggregated Hazards (Specific Effects)

First, the study includes individual hazard types simultaneously in a single model to identify their independent contributions to housing quality:

| vi=α+h∈{EQ,ST,FL}βhlnHazardi,h+1+Xi'Γ+δr+ϵi | ( SEQ Equation \* ARABIC 2) |
| --- | --- |

where h denotes earthquakes (EQ), storms (ST), and floods (FL). Xi' is variables from Phase 1 controls (poverty gap, log GDP per capita, urban population share, Government Effectiveness, and BQCI) with associated coefficients. Γ represents the vector of coefficients for these control variables, and δr denotes regional fixed effects. Including all three hazard types simultaneously allows identification of the independent contribution of each, controlling for the geographical co-occurrence of multiple hazard exposures.

### Model B: Aggregate Hazards (Total Exposure)

Model B tests whether aggregate disaster frequency,  irrespective of hazard type, catalyzes structural housing improvements:

| vi=α+βlnHazardi,h+1+Xi'Γ+δr+ϵi | ( SEQ Equation \* ARABIC 3) |
| --- | --- |

Comparing Models A and B directly tests whether hazard type matters: if disaggregated coefficients diverge while the aggregate is insignificant, the findings support hazard-differentiated DRR investment strategies; the reverse would suggest that aggregate disaster frequency alone is sufficient. For both specifications, the nested approach from Phase 1 is followed, sequentially adding controls to assess the stability of disaster learning coefficients as economic and governance factors are introduced.

## Phase 3: Heterogeneous Effects (Interaction Models)

The theoretical framework predicts that disaster learning is moderated by poverty, income, and governance quality. To test these predictions, interaction models are estimated for each hazard type and each development stage:

| vi=α+β1Disasteri,h+β2Zi+γDisasteri,h×Zi+δr+ϵi | ( SEQ Equation \* ARABIC 4) |
| --- | --- |

Where Zi represents one of the four contextual moderators: (i) Poverty Gap, (ii) log GDP per capita, (iii) Government Effectiveness, and (iv) BQCI. All continuous variables are mean-centered to facilitate the interpretation of the main effects. The coefficient of interest is γ, which captures how the marginal effect of disaster experience on housing quality varies across different levels of Zi.

# Results

Results are organized into four sections. Associations between housing safety constraints and development stage suggest a fundamental shift from poverty-dominated determinants (Stage 1) to construction-sector governance (Stage 2). Earthquake exposure is consistently associated with structural improvement across both stages, while flood and storm exposure patterns are consistent with a distinct poverty-mediated reconstruction mechanism. These patterns demonstrate robust consistency across a range of specifications, samples, and measurement approaches. Complete regression output for all specifications is reported in Appendix C.

## Baseline Models: Economic Constraints and Institutional Capacity

Table 2  Baseline Determinants of Housing Resilience

| Dependent Variable | Non-Fragile |  |  | Robust Only |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Variable | M2 | M3 | M6 | M2 | M3 | M6 |
| Poverty Gap ($4.20/day) | -0.0134*** | -0.0075** | -0.0072** | -0.0100*** | -0.0028 | -0.0019 |
|  | (0.0027) | (0.0032) | (0.0032) | (0.0031) | (0.0035) | (0.0033) |
| Poverty Gap Squared | 0.0003*** | 0.0002*** | 0.0002** | 0.0002*** | 0.0001 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  | 0.0700*** | 0.0210 |  | 0.0851*** | 0.0393 |
|  |  | (0.0239) | (0.0339) |  | (0.0255) | (0.0301) |
| Urban Population % |  |  | 0.0002 |  |  | 0.0018* |
|  |  |  | (0.0012) |  |  | (0.0010) |
| Government Effectiveness |  |  | 0.0484* |  |  | 0.0167 |
|  |  |  | (0.0266) |  |  | (0.0210) |
| Building Quality Index |  |  | 0.0080 |  |  | 0.0094** |
|  |  |  | (0.0053) |  |  | (0.0044) |
| Region FE: Middle East & North Africa | 0.0644 | 0.0536 | 0.0620 | 0.2544*** | 0.2413*** | 0.1933** |
|  | (0.0722) | (0.0679) | (0.0734) | (0.0899) | (0.0874) | (0.0910) |
| Region FE: Latin America & Caribbean | 0.0881 | 0.0743 | 0.1128** | -0.1500** | -0.1668*** | -0.1657*** |
|  | (0.0586) | (0.0572) | (0.0537) | (0.0590) | (0.0554) | (0.0574) |
| Region FE: Europe & Central Asia | -0.0455 | -0.1027** | -0.0849* | 0.1781*** | 0.1085 | 0.1110 |
|  | (0.0511) | (0.0511) | (0.0505) | (0.0657) | (0.0675) | (0.0679) |
| Region FE: North America | 0.2009*** | 0.1059* | 0.0982* | -0.2559*** | -0.3716*** | -0.3946*** |
|  | (0.0508) | (0.0641) | (0.0552) | (0.0607) | (0.0734) | (0.0765) |
| Region FE: Sub-Saharan Africa | -0.2552*** | -0.2682*** | -0.2672*** | -0.0249 | -0.0408 | -0.0587 |
|  | (0.0547) | (0.0552) | (0.0515) | (0.0542) | (0.0543) | (0.0526) |
| Region FE: South Asia | -0.2628** | -0.2375** | -0.2417** | 0.0755 | 0.1063 | 0.1113 |
|  | (0.1076) | (0.1025) | (0.0951) | (0.1120) | (0.1050) | (0.1046) |
| Constant | 0.5512*** | 0.5879*** | 0.5793*** | 0.2064*** | 0.2511*** | 0.2639*** |
|  | (0.0402) | (0.0416) | (0.0395) | (0.0478) | (0.0492) | (0.0508) |
| Observations | 150 | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.6190 | 0.6409 | 0.6539 | 0.5513 | 0.5953 | 0.6163 |
| Adj. R-squared | 0.5974 | 0.6178 | 0.6236 | 0.5259 | 0.5693 | 0.5827 |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Table 2 presents baseline results for 150 countries, revealing a shift in the determinants of structural housing safety across development stages, consistent with the two-stage framework and the DRR governance gap identified in Section 1.2.

### Stage 1: Eliminating Fragile Housing

Household poverty is the prevailing Stage 1 constraint. The poverty gap demonstrates a robust negative correlation with the non-fragile housing share (M2: −0.0134, p < 0.01), accompanied by a significant positive quadratic term, suggesting diminishing effects at extreme deprivation levels. This finding aligns with the hypothesis that housing is uniformly fragile at the deepest poverty levels, irrespective of marginal variations. This nonlinearity implies that poverty-reduction interventions yield the largest structural safety gains in moderately poor countries rather than the most deprived settings.

National income is independently and positively associated with structural safety (M3: 0.0700, p<0.01) distinct from the poverty effect. This finding indicates that national wealth and household deprivation are distinct constraints, as both variables remain significant simultaneously.

In the full specification (M6), Government Effectiveness reaches marginal significance (p < 0.10), while the BQCI remains insignificant. This finding indicates that general institutional capacity, not construction-sector-specific regulation, is the operative governance constraint at Stage 1. The coefficient of GDP exhibits a precipitous decline when governance is incorporated, indicating that income effects at this stage are partially mediated through institutional capacity. The poverty gap remains statistically significant throughout the nested specifications, consistent with its role as a constraint on eliminating fragile construction that is not fully explained by governance and income.

### Stage 2: Achieving Robust Construction

The robust-only models demonstrate a remarkable transition in constraint structure. Poverty loses all statistical significance, including the quadratic term, thereby confirming that household financial constraints do not bind at Stage 2. The GDP coefficient strengthens relative to Stage 1, consistent with the transition to engineered construction standards requiring a more extensive technical infrastructure, such as  formal construction markets, certified engineering labor, and quality materials, characteristic of higher income levels.

Construction-sector-specific institutions emerge as the dominant governance constraint at Stage 2. The BQCI attains substantial significance in M6 for robust housing (0.0094, p < 0.05), while Government Effectiveness becomes non-significant, a direct reversal from Stage 1. This finding has precise policy implications: in middle- and higher-income countries, the effective lever for improving structural safety is construction-sector regulatory capacity, such as inspection systems, professional certification, and code enforcement, not broad governance reform. The urban population share attains marginal significance at Stage 2 but not at Stage 1, consistent with the tendency of engineered construction to concentrate in formal urban markets where regulatory enforcement infrastructure is more developed.

### Regional Heterogeneity

Regional fixed effects reveal the persistent geographic disparities that remain unexplained by income, poverty levels, governance, and disaster exposure. Sub-Saharan Africa and South Asia exhibit considerably lower proportions of non-fragile housing compared to East Asia and the Pacific, even after controlling for relevant covariates. This observation suggests that construction traditions and historical regulatory development may independently influence structural outcomes, irrespective of the measured quality of governance. These residuals suggest that the regulatory history of the built environment warrants investigation as an explanatory factor beyond the governance indicators available at global scale.

## Disaster Learning and Heterogeneous Effects

Table 3  Disaster Learning and Interactions

|  | Non-fragile |  |  | Robust only |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Variable | D5 | Int. Pov | Int. WGI | D5 | Int. Pov | Int. WGI |
| Poverty Gap ($4.20/day) | -0.0065* | -0.0071** | -0.0073** | -0.0012 | -0.0022 | -0.0027 |
|  | (0.0036) | (0.0033) | (0.0033) | (0.0037) | (0.0035) | (0.0033) |
| Poverty Gap Squared | 0.0001** | 0.0001 | 0.0002** | 0.0001 | 0.0000 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) | 0.0280 | 0.0651** | 0.0349 | 0.0459 | 0.0621* | 0.0543* |
|  | (0.0360) | (0.0293) | (0.0357) | (0.0308) | (0.0331) | (0.0311) |
| Urban Population % | 0.0005 | 0.0003 | 0.0003 | 0.0021** | 0.0019* | 0.0017* |
|  | (0.0012) | (0.0012) | (0.0011) | (0.0010) | (0.0010) | (0.0010) |
| Government Effectiveness (WGI) | 0.0403 |  | 0.0518* | 0.0094 |  | 0.0171 |
|  | (0.0282) |  | (0.0285) | (0.0205) |  | (0.0220) |
| Building Quality Index | 0.0065 |  |  | 0.0096* |  |  |
|  | (0.0064) |  |  | (0.0054) |  |  |
| Earthquake Count (Log) | 0.0579*** | 0.0468*** | 0.0557*** | 0.0369* | 0.0315* | 0.0346* |
|  | (0.0180) | (0.0170) | (0.0182) | (0.0211) | (0.0179) | (0.0182) |
| Storm Count (Log) | 0.0043 | 0.0106 | 0.0042 | -0.0051 | -0.0007 | -0.0026 |
|  | (0.0158) | (0.0167) | (0.0159) | (0.0144) | (0.0147) | (0.0136) |
| Flood Count (Log) | -0.0312 | -0.0183 | -0.0206 | -0.0235 | -0.0059 | -0.0061 |
|  | (0.0201) | (0.0157) | (0.0185) | (0.0178) | (0.0152) | (0.0156) |
| Poverty × Earthquake |  | -0.0041*** |  |  | -0.0028** |  |
|  |  | (0.0012) |  |  | (0.0013) |  |
| Poverty × Storm |  | 0.0012* |  |  | 0.0008 |  |
|  |  | (0.0007) |  |  | (0.0007) |  |
| Poverty × Flood |  | 0.0033*** |  |  | 0.0028** |  |
|  |  | (0.0012) |  |  | (0.0012) |  |
| Gov. Effectiveness × Earthquake |  |  | 0.0349** |  |  | 0.0447** |
|  |  |  | (0.0172) |  |  | (0.0185) |
| Gov. Effectiveness × Storm |  |  | -0.0312** |  |  | -0.0314** |
|  |  |  | (0.0147) |  |  | (0.0135) |
| Gov. Effectiveness × Flood |  |  | 0.0054 |  |  | -0.0119 |
|  |  |  | (0.0172) |  |  | (0.0150) |
| Constant | 0.5548*** | 0.5713*** | 0.5484*** | 0.2594*** | 0.2568*** | 0.2477*** |
|  | (0.0435) | (0.0403) | (0.0395) | (0.0531) | (0.0534) | (0.0499) |
| Region FE | Yes | Yes | Yes | Yes | Yes | Yes |
| Observations | 150 | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.6758 | 0.6797 | 0.6866 | 0.6288 | 0.6296 | 0.6439 |
| Adj. R-squared | 0.6395 | 0.6411 | 0.6462 | 0.5873 | 0.5851 | 0.5981 |

Note: *** p<0.01, ** p<0.05, * p<0.10.

The paper tests whether disaster exposure is associated with housing improvements (Phase 2) and whether these associations vary systematically across contexts (Phase 3). The results (Table 3) reveal pronounced, hazard-specific patterns, with significant heterogeneous effects.

### Main Disaster Effects: Hazard-Specific Learning

Across all specifications, earthquake exposure is associated with systematic housing quality improvements and maintains this association. In the full policy model (D5), earthquake count exhibits a highly significant coefficient for non-fragile housing. This association persists after controlling for poverty, GDP, urbanization, governance, and building regulations, suggesting it is not fully accounted for by structural development factors alone. For Stage 2 outcomes, earthquakes maintain marginal significance in D5 and regain full significance in the governance interaction model. This indicates that earthquake learning operates across both housing quality stages, though with somewhat weaker effects at Stage 2 where institutional capacity becomes more binding.

Flood and storm exposure show no significant main effects on housing quality. Storm exposure exhibits near-zero coefficients, while flood exposure shows consistently negative though insignificant associations. The flood pattern suggests repeated flood exposure may concentrate vulnerable populations without triggering housing upgrading, while storms affect temporary roof damage rather than structural integrity. The result may reflect that, unlike earthquakes that require strengthening of each building, flood and storms may trigger other resilience measures, such as urban planning and flood control infrastructures. These divergent patterns strongly support hazard-specific rather than combined disaster modeling.

Model B (Table A-7 and A-8, Panel D) confirms that aggregate disaster exposure shows no significant association with structural housing safety, regardless of hazard type (non-fragile: 0.0042, insignificant; robust-only: −0.0103, insignificant). This null result is analytically significant because it demonstrates that the earthquake learning effect identified in Model A is masked when disaster exposure is aggregated across hazard types. This directly validates the hazard-disaggregated modeling approach and confirms that DRR research relying on composite disaster exposure metrics will systematically understate the structural safety benefits of seismic experience.

### Poverty and Income Interactions: Reconstruction Leap

The interaction models demonstrate that the impact of poverty on disaster learning exhibits contradictory trends, contingent on the nature of the hazard. The Poverty × Earthquake interaction is significantly negative for both Stage 1 (−0.0041, p<0.01) and Stage 2 (−0.0028, p<0.05). These findings confirm that translating seismic experience into structural improvement requires a minimum threshold of construction sector capacity, which is unavailable in the poorest settings. The Poverty × Flood interaction is significantly positive for both stages, and Poverty × Storm is positive and marginally significant for Stage 1, consistent with a Reconstruction Leap mechanism. In high-poverty settings, flood and storm exposure is associated with disproportionate reductions in the most fragile housing stock, precisely where baseline quality is lowest. This pattern is consistent with post-disaster reconstruction programs generating humanitarian pressure toward minimum building standards, though the mechanism cannot be directly observed in cross-national data at this scale.

### Governance Interactions: Institutional Capacity as Enabler

The quality of governance exerts a substantial moderating influence on seismic learning. The Government Effectiveness × Earthquake interaction is positive and significant at both Stage 1 (0.0349, p < 0.05) and Stage 2 (0.0447, p < 0.05), consistent with better-governed countries showing a stronger association between seismic experience and structural housing improvement. This finding is consistent with institutional capacity enabling sustained building code reform beyond immediate post-disaster response. The interaction strengthens at Stage 2, consistent with the baseline finding that construction-sector governance is the binding constraint at higher resilience thresholds, directly relevant to targeting Sendai Priority 3 investments.

The Government Effectiveness × Storm interaction, which is significantly negative (β = −0.0312, p < 0.05), warrants careful interpretation. The most plausible mechanism is that well-governed states prioritize macro-level hydrometeorological infrastructure, such as drainage systems, embankments, and early warning systems, over building-level reinforcement measures. Storms can cause damage to facades and roofs that may not be registered as structural typology change in the GEM taxonomy. The coexistence of these patterns substantiates the notion that the nature of hazard directly correlates with the governance channel through which disaster learning functions.

## The Income Heterogeneity: Validating the Two-Stage Framework

To test the two-stage framework directly, the study estimates the baseline specification separately for low/lower-middle income countries (below median GDP per capita of $14,885 PPP, n=75) and upper-middle/high income countries (above median, n=75).

Results (Table 4) provide validation of this framework. In lower-income countries, the poverty gap is a Stage 1 constraint that is highly significant, with a confirmed nonlinear effect. The BQCI reaches significance for Stage 2 (p<0.05), confirming that the regulatory capacity of the construction sector is important for achieving engineered standards even in environments with limited resources. In higher-income countries, however, poverty metrics become entirely insignificant; instead, government effectiveness and earthquake exposure emerge as the dominant predictors. The substantially higher R² for Stage 2 (0.59–0.60) than Stage 1 (0.37–0.39) in higher-income settings suggests that the transition to robust engineered construction is more systematically associated with measurable institutional variables than the Stage 1 transition, consistent with Stage 2 being a regulatory rather than purely economic process. The persistent negative association between flood exposure and housing quality in lower-income sub-samples warrants further investigation.

Table 4  Income Heterogeneity and the Two-Stage Framework

|  | Lower- Income |  |  |  | Higher Income |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) |
| Dependent Variable: | Non-fragile | Robust only | Non-fragile | Robust only | Non-fragile | Robust only | Non-fragile | Robust only |
| Poverty Gap | -0.006*** | -0.002 | -0.006*** | -0.002 | -0.034 | -0.033 | -0.039 | -0.030 |
|  | (0.002) | (0.003) | (0.002) | (0.003) | (0.033) | (0.029) | (0.038) | (0.029) |
| Poverty Gap Squared | 0.000*** | 0.000 | 0.000** | 0.000 | 0.004 | 0.004 | 0.003 | 0.003 |
|  | (0.000) | (0.000) | (0.000) | (0.000) | (0.003) | (0.002) | (0.003) | (0.002) |
| Log GDP PC | 0.067* | 0.030 |  |  | 0.119* | 0.006 |  |  |
|  | (0.039) | (0.040) |  |  | (0.070) | (0.056) |  |  |
| Urban Pop % | 0.002* | 0.001 | 0.003** | 0.002 | -0.002 | 0.002* | -0.002 | 0.002 |
|  | (0.001) | (0.001) | (0.001) | (0.001) | (0.002) | (0.001) | (0.002) | (0.001) |
| Gov Effectiveness |  |  | 0.038 | -0.018 |  |  | 0.082* | 0.030 |
|  |  |  | (0.030) | (0.032) |  |  | (0.043) | (0.029) |
| BQCI |  |  | 0.007 | 0.011** |  |  | 0.005 | -0.004 |
|  |  |  | (0.006) | (0.005) |  |  | (0.017) | (0.012) |
| Earthquake | 0.089*** | 0.047* | 0.078*** | 0.035 | 0.081*** | 0.060** | 0.075*** | 0.060* |
|  | (0.032) | (0.028) | (0.030) | (0.026) | (0.024) | (0.030) | (0.024) | (0.031) |
| Storm | 0.022 | -0.001 | 0.021 | -0.003 | -0.022 | -0.018 | -0.025 | -0.025 |
|  | (0.017) | (0.018) | (0.018) | (0.018) | (0.030) | (0.026) | (0.029) | (0.026) |
| Flood | -0.053* | -0.021 | -0.052* | -0.027 | -0.032 | -0.043* | -0.021 | -0.032 |
|  | (0.029) | (0.028) | (0.030) | (0.029) | (0.023) | (0.024) | (0.030) | (0.028) |
| Constant | 0.399*** | 0.062 | 0.417*** | 0.103* | 0.758*** | 0.501*** | 0.709*** | 0.501*** |
|  | (0.050) | (0.049) | (0.049) | (0.054) | (0.074) | (0.107) | (0.076) | (0.097) |
| Region FE | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Observations | 75 | 75 | 75 | 75 | 75 | 75 | 75 | 75 |
| R-squared | 0.757 | 0.510 | 0.757 | 0.527 | 0.378 | 0.597 | 0.390 | 0.603 |
| Adj. R-squared | 0.710 | 0.415 | 0.706 | 0.426 | 0.257 | 0.519 | 0.260 | 0.519 |

Notes: *** p<0.01, ** p<0.05, * p<0.10.

## External Validation Using Independent Data

The results using MPI as an independent variable, presented in Appendix Table A-5, provide strong external validity for the main findings. All core relationships are replicated with theoretically consistent signs and notably higher explanatory power (R2 ranging from 0.74 to 0.79). Specifically, the positive and significant coefficients for GDP per capita and governance effectiveness persist. The improved model fit in the MPI specifications likely stems from the fact that the MPI’s focus on basic material adequacy, such as durable walls and weather-resistant roofing, is more directly responsive to income levels and institutional capacity than the broader engineering-based classifications found in the GEM taxonomy.

Crucially, the hazard-specific learning hypothesis is also confirmed. The effect of earthquake risk remains positive and significant, while flood risk exerts a consistently negative and significant influence on housing quality. These results reinforce the theoretical distinction between these two hazards: earthquake resilience is primarily embedded within the structural robustness of individual buildings, thereby incentivizing better construction materials. In contrast, flood risk is often managed through collective physical protection (e.g., dikes) or land-use planning, which may not directly translate into improved building-level material quality.

In addition to the MPI analysis, unit-weighted housing proportions are tested instead of the cost-weighted baseline. GDP, poverty gap, and government effectiveness show stable results, confirming that fundamental development-housing relationships are robust to aggregation choices. Disaster effects reveal important distributional patterns, particularly for earthquake resilience. The earthquake frequency effect is statistically significant but weakened under unit-weighting. This suggests that earthquake-resilient construction is disproportionately concentrated in higher-cost, formal housing sectors where specialized structural engineering can be effectively implemented. Lower-cost and informal housing, which comprises the majority of dwellings in earthquake-prone developing countries, shows substantially less evidence of seismic adaptation, despite facing similar or greater exposure. This finding has significant equity implications: aggregate improvements in building quality measured by economic value may mask persistent vulnerabilities among poorer populations, who remain disproportionately at risk in seismic zones even as national-level housing indices improve.

## Alternative disaster measures

Alternative disaster measures (Table A-4) test whether occurrence, impact, or frequency drives learning. The population normalized disaster count (Relative Exposure - Panel A) aligns with the main findings, where earthquakes strengthen housing resilience at the robust-only tier.  The magnitude of the coefficient is actually larger than the baseline. This reinforces the core thesis: countries with higher relative exposure to earthquakes invest more in housing quality.

Binary disaster indicators (any vs. none – Panel B) show substantially weaker effects than continuous specifications, with disaster variables becoming non-significant. This suggests that simply "having experienced an earthquake" is insufficient to drive national-level improvements in housing vulnerability. The mortality-based specifications (Panel C) confirms that seismic learning is not merely a trauma-response to high death tolls. Instead, as the main results and Panel A suggest, it is the frequency of events that matters. For storms, however, the significant mortality effect suggests that wind-related housing improvements may be more reactive to high-casualty events compared to seismic policies.

## Additional Robustness Checks

Results prove stable across extensive robustness checks (Tables A-2 through A-6). Alternative poverty specifications (Table A-2) confirm findings across functional forms and poverty line definitions; effects weaken at extreme poverty thresholds, consistent with housing being uniformly fragile at deep deprivation levels regardless of marginal variation. Sample restrictions excluding small island states and outlier disaster-exposure countries (Table A-3) leave core results unchanged, confirming that the earthquake association reflects accumulated event frequency rather than a single catastrophic event. Jackknife analysis (Table A-6) confirms that no single geographic cluster drives the earthquake or poverty findings.

# Discussion

The two-stage constraint shift identified in this study, where household poverty governing Stage 1 progress and construction-sector governance governing Stage 2, has a precise implication for DRR investment design. This finding is inconsistent with the prevailing assumption that economic growth and general governance improvement will, over time, automatically produce safer housing stocks. The significance of government effectiveness is only evident at Stage 1, but not at Stage 2. A shift in significance to BQCI at Stage 2, suggesting that the institutional bottleneck is not indicative of generic state capacity but rather of the specific regulatory infrastructure that governs the construction process, the inspection thereof, and the professional accountability for structural outcomes. In lower-income contexts, the absence of economic enabling conditions, such as secure tenure, accessible materials markets, and poverty-reduction support, hinders the adoption of building codes [42] [43]. In middle- and higher-income countries, it is evident that income growth without the development of regulatory capacity in the construction sector will not exceed the Stage 2 threshold, irrespective of the quality of general governance [44] [45]. This staged sequencing directly informs Sendai Priority 3's investment framework, providing a greater level of granularity regarding the specific investments and their respective development stages [5]. This level of detail is currently not captured by aggregate DRR investment targets [46].

The hazard-specific learning findings carry equally significant implications. Earthquake exposure is consistently associated with structural improvement across both stages and income groups, with stronger associations in well-governed countries. This pattern is consistent with a building regulatory channel through which seismic experience may be encoded into enforceable standards. This finding aligns with the historical pattern whereby major seismic events catalyze specific legislative and code enforcement reforms rather than broad governance change [19]. This seismic learning pathway is contingent upon institutional capacity to encode disaster lessons into enforceable construction standards beyond the immediate post-disaster window [47] [48] [49]. The Reconstruction Leap pattern, in which flood and storm exposure is associated with structural upgrading in high-poverty contexts, is consistent with a different institutional pathway, plausibly through post-disaster reconstruction programs tied to minimum construction standards. Indeed, post-disaster needs assessments have increasingly incorporated Build Back Better principles, resulting in the generation of grant programs tied to minimum construction standards. [50] However, it falls short in areas where governance and construction capacities are lacking [51]. The enhancement of pre-arranged DRM financing instruments has been demonstrated to mitigate the humanitarian speed-quality trade-off that propels structurally suboptimal post-flood reconstruction in the most resource-constrained settings [52]. Taken together, these patterns support the proposition that hazard type shapes the governance mechanisms through which resilience improvements occur, a distinction that DRR program design must reflect.

The persistent regional fixed effects deserve interpretation beyond statistical residuals. Sub-Saharan Africa and South Asia underperform relative to their income and governance indicators in ways consistent with a specific and well-documented regulatory failure: widespread code adoption without enforcement infrastructure [44]. Most Sub-Saharan African countries have formal building codes, many inherited from colonial regulatory frameworks, but lack the inspection personnel, municipal capacity, and professional certification systems to implement them at scale [53]. South Asia's underperformance likely reflects the dominance of informal rural construction that occurs entirely outside formal regulatory systems, where building permits are rarely sought and inspections are absent [54]. The profile of Latin America and the Caribbean, marked by reasonable Stage 1 performance but weak Stage 2 outcomes, aligns with the broader regional pattern. The region has made notable advances in seismic code development; however, enforcement and compliance gaps remain substantial, and flood hazard integration is largely approached reactively [55]. These regional patterns corroborate the paper's central finding from an independent angle: no existing cross-national governance indicator captures the gap between regulatory intent and construction practice. Addressing this gap through measures such as inspector density, permit compliance rates, and professional certification coverage constitutes the most pressing data priority for future cross-national DRR research.

Three limitations bound the interpretation of these findings. The cross-sectional design cannot establish causality. While the temporal precedence of governance institutions and income over the accumulated housing stock makes reverse causation implausible as the primary channel, the possibility cannot be fully excluded without panel data. Second, national-level analysis masks within-country variation in enforcement intensity, construction market capacity, and household decision-making. Third, the GEM taxonomy classifies structural typology rather than construction quality within typology, and therefore cannot capture quality variation among buildings of the same structural class. Future research should prioritize three areas: subnational panel data linking permit records, inspection outcomes, and disaster events, to enable pre-post identification of regulatory learning mechanisms; engineering audit surveys to validate whether Reconstruction Leap gains meet structural safety standards; and integration of DRM financing instrument data to directly test whether contingent financing modality moderates post-disaster reconstruction quality.

# Conclusion

This paper provides cross-national evidence consistent with structural housing safety being governed by staged and qualitatively distinct constraint regimes. Poverty reduction and construction-sector governance are not interchangeable levers. They operate at different development stages, through different institutional channels, and require different DRR investment strategies. By providing the first cross-national empirical basis for this distinction across 150 countries, the paper offers a diagnostic framework for sequencing DRR investments under the Sendai Framework. This framework moves beyond aggregate targets toward a stage-specific and hazard-differentiated investment logic The further finding that disaster learning appears governance-mediated and hazard-specific, with seismic exposure associated with regulatory reform pathways and hydrometeorological events associated with reconstruction-led upgrading, supports the conclusion that Build Back Better outcomes are institutionally contingent, not automatic consequences of disaster exposure.

For practice, the implications are precise. Low-income countries should prioritize poverty reduction and tenure security as prerequisites for building code adoption. Middle- and higher-income countries should invest in construction-sector regulatory infrastructure rather than broad governance reform. Post-disaster programs should be designed to leverage reconstruction pressure through binding minimum standards backed by pre-arranged financing. For research, the priority is depth over breadth. Subnational data linking permit records, inspection outcomes, and reconstruction quality would transform the staged diagnostic framework proposed here from a cross-national pattern into a locally actionable tool for DRR governance.

# CRediT authorship contribution statement

Yasuhiro Kawasoe: Conceptualization, Data curation, Formal analysis, Methodology, Software, Validation, Visualization, Writing – original draft

# Acknowledgments

The author produced the paper as a part of the efforts under the Resilient Housing Thematic Area of the Global Facility for Disaster Reduction and Recovery (GFDRR). The author gratefully acknowledges valuable comments and suggestions from Catherine Lynch (Senior Urban Specialist), Karima Ben Bih (Senior Disaster Risk Management Specialist), Alvina Elisabeth Erman (Economist), Chloe Genevieve Helene Desjonqueres (Climate Finance Specialist), Solene Adeline Johanne Masson (Consultant), Luis Miguel Triveno Chan Jan (Senior Urban Development Specialist), Bramka Arga Jafino (Disaster Risk Management Specialist), and Robin Middelanis (Consultant). The author received guidance from Niels Holm-Nielsen (Practice Manager) and Ana Campos Garcia (Lead Disaster Risk Management Specialist).

The author used generative AI tools to assist with coding and editing. All text was reviewed and verified by the author, who takes full responsibility for the content and any remaining errors.

This paper was made possible with the financial support of the Japan-World Bank Program for Mainstreaming Disaster Risk Management in Developing Countries, which is financed by the Government of Japan and receives technical support from the World Bank Tokyo Disaster Risk Management Hub.

The findings, interpretations, and conclusions expressed in this paper are entirely those of the author. They do not necessarily represent the views of the International Bank for Reconstruction and Development/World Bank and its affiliated organizations, or those of the Executive Directors of the World Bank or the governments they represent. The World Bank does not guarantee the accuracy of the data included in this work.

# References

| [1] | Cabinet Office, Government of Japan, "令和６年能登半島地震に係る被害状況等について," 15 10 2025. [Online]. Available: https://www.bousai.go.jp/updates/r60101notojishin/r60101notojishin/pdf/r60101notojishin_59.pdf. |
| --- | --- |
| [2] | J. Brand, K. D. Cox, J. E. Daniell, S. Esper, R. Gunasekera, O. A. Ishizawa Escudero, B. Khazai, J. L. D. C. Macabuag, S. G. Otálora, A. Pomonis and R. A. Romero Hernandez, "Global Rapid Post-Disaster Damage Estimation (GRADE) Report : February 6, 2023 Kahramanmaraş Earthquakes - Türkiye Report," World Bank Group., Washington, D.C., 2023. |
| [3] | World Bank, "Earthquake Direct Damage in Syria Estimated at $5.1 billion in Areas Already Severely Ravaged by Long Conflict and Displacement," 28 2 2023. [Online]. Available: https://www.worldbank.org/en/news/press-release/2023/02/28/earthquake-direct-damage-in-syria-estimated-at-5-1-billion-in-areas-already-severely-ravaged-by-long-conflict-and-displa. |
| [4] | World Bank; GFDRR; Inter-American Development Bank., A Global Assessment of Building Codes: Current Status and Evolving Needs to Promote Resilient, Green and Inclusive Buildings., World Bank and the Inter-American Development Bank., 2025. |
| [5] | UNDRR, "Sendai Framework for Disaster Risk Reduction 2015–2030.," United Nations Office for Disaster Risk Reduction., 2015. |
| [6] | UN-HABITAT, "2024 Annual Report: Adequate Housing for All,," UN-HABITAT, 2025. |
| [7] | UN-Habitat, "World Cities Report: Envisaging the Future of Cities," United Nations Human Settlements Programme., 2022. |
| [8] | UNDRR, "Global Assessment Report on Disaster Risk Reduction," 2025. |
| [9] | J. Friberg, "When the poor lose even more: A framework of normalized measures for disaster impacts and household economic vulnerability," World Development Sustainability, vol. 1, 2022. |
| [10] | M. D. Silva and A. Kawasaki, "Socioeconomic Vulnerability to Disaster Risk: A Case Study of Flood and Drought Impact in a Rural Sri Lankan Community," Ecological Economics, vol. 152, pp. 131-140, 2018. |
| [11] | O. Nielsen and M. Sanders, "Why affordable housing is more vulnerable to disasters," 21 October 2024. [Online]. Available: https://www.preventionweb.net/drr-community-voices/why-affordable-housing-more-vulnerable-disasters. |
| [12] | O. Cohen, S. McTarnaghan and A. Junod, "Preserving, Protecting, and Building Climate-Resilient Affordable Housing," Urban Institute, 2024. |
| [13] | C. Wang, S. E. Antos and L. M. Triveno, "Automatic detection of unreinforced masonry buildings from street view images using deep learning-based image segmentation," Automation in Construction, vol. 132, 2021. |
| [14] | N. Baum-Snow and G. Duranton, "Chapter 6 - Housing supply and housing affordability," Handbook of Regional and Urban Economics, pp. 353-461, 2025. |
| [15] | C. Kenny, "Why Do People Die in Earthquakes? The Costs, Benefits and Institutions of Disaster Risk Reduction in Developing Countries," World Bank Policy Research Working Paper, 2016. |
| [16] | M. Bangalore, S. Hallegatte, A. Vogt-Schilb and J. Rozenberg, "Unbreakable: Building the Resilience of the Poor in the Face of Natural Disasters," World Bank, 2017. |
| [17] | P. Keefer, E. Neumayer and T. Plümper, "Earthquake Propensity and the Politics of Mortality Prevention," World Development, vol. 39, no. 9, pp. 1530-1541, 2011. |
| [18] | M. Hube, A. Marihuén, J. d. l. Llera and B. Stojadinovic, "Seismic behavior of slender reinforced concrete walls," Engineering Structures, vol. 80, pp. 377-388, 2014. |
| [19] | T. Moullier and K. Sakoda, "Building regulation for resilience : converting disaster experience into a safer built environment - the case of Japan (English)," World Bank Group, Washington, D.C., 2018. |
| [20] | R. K. Shrestha, H. R. Parajuli and B. Poudel, "Building typologies and code compliance issues of reconstructed houses after 2015 Mw 7.8 Gorkha, Nepal earthquake: Experience from Dhading District earthquake,," Progress in Disaster Science, vol. 10, 2021. |
| [21] | R. Yore, J. F. Walker and A. Suppasri, "Transitions to recovery: The shelter and housing journey following disaster," International Journal of Disaster Risk Reduction, vol. 128, 2025. |
| [22] | H. D. Chavez and S. Lim, "Disaster policy change through the enactment of the National Disaster Risk Reduction and Management law in the Philippines: Punctuated equilibrium and multiple streams theories," International Journal of Disaster Risk Reduction, vol. 125, 2025. |
| [23] | D. Sanderson, F. Perugia, A. Kroen, E. Liu and J. Dodson, "Inquiry into housing policy and disaster: better coordinating actors, responses and data," AHURi, 2025. |
| [24] | N. Ambraseys and R. Bilham, "Corruption kills.," Nature, vol. 469, p. 153–155, 2011. |
| [25] | S. Cevik and J. T. Jalles, "Corruption Kills: Global Evidence from Natural Disasters," IMF Working Papers , 2023. |
| [26] | World Bank, "Indonesia Economic Prospects : People-First Housing - A Roadmap from Homes to Jobs to Prosperity in Indonesia," World Bank Group, Washington D.C., 2025. |
| [27] | B. Dasgupta, S. V. Lall, N. L. Gracia, D. Mason, J. I. Parby and C. Young, "Stocktaking of the housing sector in Sub-Saharan Africa : challenges and opportunities.," World Bank Group., Washington, D.C., 2015. |
| [28] | S. Wahba, L. Triven, H. C. Terraza and S. E. Antos, "Building better before the next disaster: How retrofitting homes can save lives and strengthen economies," 9 October 2018. [Online]. Available: https://blogs.worldbank.org/en/sustainablecities/building-better-next-disaster-how-retrofitting-homes-can-save-lives-and-strengthen-economies. |
| [29] | K. W. Deininger and A. Goyal, "Land Policies and Institutions for Equitable and Resilient Growth in Africa," World Bank Group, Washington D.C., 2024. |
| [30] | C. C. A. C. C. C. H. D. J. H. M. M. L. P. N. R. A. S. V. Yepes-Estrada, "Global Building Exposure Model for Earthquake Risk Assessment.," Earthquake Spectra, 2023. |
| [31] | V. Silva, D. Amo-Oduro, A. Calderon, C. Costa, J. Dabbeek, V. Despotaki, L. Martins, M. Pagani, A. Rao, M. Simionato, D. Viganò, C. Yepes-Estrada, A. Acevedo, H. Crowley, N. Horspool, J. Kishor , M. Journeay and M. Pittore, "Development of a global seismic risk model," Earthquake Spectra, vol. 36, pp. 372-394, 2020. |
| [32] | R. Middelanis, B. A. Jafino, R. Hill, M. C. Nguyen and S. Hallegatte, "Global Socio-economic Resilience to Natural Disasters.," World Bank Policy Research Working Paper, 2025. |
| [33] | S. M. Zahrai and M. Heidarzadeh, "Destructive effects of the 2003 bam earthquake on structures.," Asian Journal of Civil Engineering, vol. 8, pp. 329-342, 2007. |
| [34] | D. Gautam and H. Chaulagain, "Structural performance and associated lessons to be learned from world earthquakes in Nepal after 25 April 2015 (MW 7.8) Gorkha earthquake," Engineering Failure Analysis, vol. 68, pp. 222-243, October 2016. |
| [35] | E. Işık, "Structural Failures of Adobe Buildings during the February 2023 Kahramanmaraş (Türkiye) Earthquakes," Applied Sciences, August 2023. |
| [36] | A. Chourasia, S. Bhattacharyya, P. Bhargava and N. Bhandari, "Influential aspects on seismic performance of confined masonry construction.," Natural Science, vol. 5, no. 8A1, pp. 56-62, 2013. |
| [37] | A. Chourasia, S. Bhattacharyya, N. Bhandari and P. Bhargava, "Seismic Performance of Different Masonry Buildings: Full-Scale Experimental Study," Journal of Performance of Constructed Facilities, 2016. |
| [38] | J. J. S. B. E. F. E. C. S. G. D. Q. a. M. T. R. Pérez Gavilán Escalante, "Experimental Research Studies on Seismic Behaviour of Confined Masonry Structures: Current Status and Future Needs," Buildings, vol. 13, 2023. |
| [39] | E. M. Foster, D. M. Jolliffe, G. Lara Ibarra, C. Lakner and S. K. Tetteh Baah, "Global Poverty Revisited Using 2021 PPPs and New Data on Consumption," Policy Research Working Paper, 2025. |
| [40] | D. Kaufmann and A. C. Kraay, "The Worldwide Governance Indicators : Methodology and 2024 Update (English)," Policy Research working paper;, 2024. |
| [41] | D. Delforge, V. Wathelet, R. Below, C. L. Sofia, M. Tonnelier, J. A. v. Loenhout and N. Speybroeck, "EM-DAT: the Emergency Events Database," International Journal of Disaster Risk Reductio, vol. 124, 2025. |
| [42] | S. Hallegatte, A. Vogt-Schilb, J. Rozenberg, M. Bangalore and C. Beaudet, "From Poverty to Disaster and Back: a Review of the Literature," Economics of Disasters and Climate Change, vol. 4, p. 223–247, 2020. |
| [43] | C. L. T. B. A. F. A. W. A. M. B. D. D. E. L. a. M. M. C. C. (. a. L. T. (. Cociña, "Better cities are possible: transforming informal settlements on a warming planet. IIED, London.," 2025. |
| [44] | World Bank Group, "Building Regulation for Resilience: Managing Risks for Safer Cities.," World Bank, 2015. |
| [45] | C. Kenny, "Construction, Corruption, and Developing Countries.," Policy Research Working Paper, 2007. |
| [46] | United Nations Office for Disaster Risk Reduction (UNDRR), "The Report of the Midterm Review of the Implementation of the Sendai Framework for Disaster Risk Reduction 2015-2030," 2023. |
| [47] | N. Giordano, R. D. Risi, J. Macdonald, K. Goda, I. Kafodya and I. Ngoma, "Implications of building code enforcement and urban expansion on future earthquake loss in East Africa: case study—Blantyre, Malawi," Natural Hazards, vol. 117, pp. 1083-1104, 2023. |
| [48] | S. Ainuddin, U. Mukhtar and S. Ainuddin, "Public perception about enforcement of building codes as risk reduction strategy for seismic safety in Quetta, Baluchistan," International Journal of Disaster Risk Reduction, pp. 99-106, 2014. |
| [49] | P. H. Moshfegh, S. V. Sharif and H. Kashani, "Assessing the impact and equitability of governance mechanisms for seismic resilience enhancement," Sustainable Cities and Society2025, vol. 134, 2025. |
| [50] | "Post-disaster needs assessment guidelines volume A," World Bank Group., Washington, D.C. , 2013. |
| [51] | E. Dube, G. Wedawatta and K. Ginige, "uilding-Back-Better in Post-Disaster Recovery: Lessons Learnt from Cyclone Idai-Induced Floods in Zimbabwe," International Journal of Disaster Risk Science, vol. 12, pp. 700-712, 2021. |
| [52] | D. J. Clarke and S. Dercon, "Dull Disasters?: How Planning Ahead Will Make a Difference," World Bank, 2016. |
| [53] | World Bank; Global Facility for Disaster Reduction and Recovery., "Building Regulations in Sub-Saharan Africa: A Status Review of the Building Regulatory Environment," World Bank, 2023. |
| [54] | I. Ahmed, T. Gajendran, G. Brewer, K. Maund, J. von Meding and J. Mackee, "Compliance to building codes for disaster resilience: Bangladesh and Nepal.," Procedia Engineering, vol. 212, pp. 986-993, 2018. |
| [55] | A. Alvear, J. P. Campos, J. Ciancio, W. Dalaison, G. De Angelis, M. A. Escovar, H. Madrid, R. Narváez, F. Pedrasa, R. Porta García, G. Suarez and A. Zambrano, "Resilience and Sustainability in Building Codes in Latin America and the Caribbean," IDB, 2023. |
| [56] | NIDM and ADPC, "Three days Online Training Program Report on Post Disaster Housing ReconstructionLessons from the Past and Future Aspects," 2021. |
| [57] | D. H. Aquino, S. Wilkinson, G. M. Raftery and R. Potangaroa, "Building back towards storm-resilient housing: Lessons from Fiji's Cyclone Winston experience," International Journal of Disaster Risk Reduction, vol. 33, pp. 355-364, 2019. |
| [58] | J. Karki, S. Matthewman and J. H. Grayman, "Nayã Ghar (A new house): Examining post-earthquake housing reconstruction issues in Nepal," International Journal of Disaster Risk Reduction, vol. 78, 2022. |
| [59] | A. Scolobig, J. Linnerooth-Bayer and M. Pelling, "Drivers of transformative change in the Italian landslide risk policy," International Journal of Disaster Risk Reduction, vol. 9, pp. 124-136, 2014. |

# Appendix A. Correlation matrix

Table A-1 shows the correlation matrix for key variables. As expected, building robustness is strongly positively correlated with GDP per capita and negatively correlated with poverty. Government effectiveness shows high correlation with GDP (r=0.796, p<0.01), which the paper addresses by testing these variables separately and jointly in the specifications.

### Table A-1: Correlation matrix

|  | (1) Robustness(M/H) | (2) Robustness(H) | (3) Poverty Gap | (4) Log GDP p.c. | (5) Gov. Effectiveness | (6) Building Quality | (11) Urban Pop % | (7) Log EQK | (8) Log STM | (9) Log FLD | (10) Log 3-Haz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) Robustness(M/H) | 1.000*** |  |  |  |  |  |  |  |  |  |  |
| (2) Robustness(H) | 0.570*** | 1.000*** |  |  |  |  |  |  |  |  |  |
| (3) Poverty Gap | -0.634*** | -0.461*** | 1.000*** |  |  |  |  |  |  |  |  |
| (4) Log GDP p.c. | 0.637*** | 0.579*** | -0.755*** | 1.000*** |  |  |  |  |  |  |  |
| (5) Gov. Effectiveness | 0.500*** | 0.480*** | -0.533*** | 0.796*** | 1.000*** |  |  |  |  |  |  |
| (6) Building Quality | 0.298*** | 0.440*** | -0.299*** | 0.423*** | 0.326*** | 1.000*** |  |  |  |  |  |
| (11) Urban Pop % | 0.531*** | 0.469*** | -0.543*** | 0.702*** | 0.505*** | 0.299*** | 1.000*** |  |  |  |  |
| (7) Log EQK | 0.270*** | 0.082 | -0.117 | 0.039 | -0.002 | 0.266*** | 0.014 | 1.000*** |  |  |  |
| (8) Log STM | 0.264*** | -0.007 | -0.124 | 0.189** | 0.203** | 0.115 | 0.128 | 0.455*** | 1.000*** |  |  |
| (9) Log FLD | -0.126 | -0.210*** | 0.208** | -0.193** | -0.248*** | 0.231*** | -0.093 | 0.550*** | 0.375*** | 1.000*** |  |
| (10) Log 3-Haz | 0.015 | -0.183** | 0.116 | -0.092 | -0.111 | 0.215*** | -0.053 | 0.663*** | 0.674*** | 0.906*** | 1.000*** |

# Appendix B. Comprehensive Robustness Checks

This appendix presents comprehensive robustness checks for the main findings. Following best practices in cross-country regression analysis, the author tests sensitivity to: (i) alternative poverty specifications, (ii) sample restrictions, (iii) outlier treatment, (iv) alternative disaster measures, (v) external validation with different data sources, (vi) interaction effects, and (vii) regional heterogeneity.

To avoid multicollinearity between GDP, governance, and building quality control index, the paper follows the main analysis strategy and estimates two specifications for each robustness check:

- Log(GDP per capita) + Poverty + Urbanization +Disasters + Regional FE

- Government Effectiveness + BQCI + Poverty + Urbanization + Disasters + Regional FE

We test both dependent variables: (i) VULN = non-fragile housing (robust + medium quality), and (ii) ONLY = robust housing only, corresponding to the two-stage housing ladder framework. Tables A-2 to A-6  provide detailed coefficient estimates for selected checks discussed in the main text.

### Table A-2: Robustness Checks - Alternative Poverty Specifications

Note: This table presents results using alternative poverty measures. Panel A uses poverty quartiles (Q4 = highest poverty quartile). Panel B uses poverty headcount ratio at $4.20/day. Panel C uses poverty gap at $3.00/day. Panel D uses poverty gap at $8.30/day. All models include regional fixed effects. Robust standard errors in parentheses. ***p<0.01, **p<0.05, *p<0.10.

| Panel A: Poverty Quartiles | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Q2 | -0.053 | -0.140*** | -0.048 | -0.139*** |
|  | (0.047) | (0.042) | (0.045) | (0.042) |
| Poverty Q3 | -0.078 | -0.183*** | -0.078 | -0.182*** |
|  | (0.055) | (0.050) | (0.056) | (0.050) |
| Poverty Q4 (Highest) | -0.146* | -0.191** | -0.156** | -0.188** |
|  | (0.077) | (0.075) | (0.077) | (0.074) |
| Log GDP PC | 0.059** | 0.028 |  |  |
|  | (0.029) | (0.029) |  |  |
| Governance Effectiveness |  |  | 0.051** | 0.012 |
|  |  |  | (0.023) | (0.021) |
| Building Quality Control Index |  |  | 0.007 | 0.008 |
|  |  |  | (0.007) | (0.005) |
| Urban Population % | 0.000 | 0.002** | 0.001 | 0.002*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.066*** | 0.049*** | 0.060*** | 0.044** |
|  | (0.018) | (0.017) | (0.018) | (0.018) |
| Log Storm count | 0.007 | -0.003 | 0.006 | -0.003 |
|  | (0.016) | (0.013) | (0.015) | (0.013) |
| Log Flood count | -0.032* | -0.014 | -0.031 | -0.020 |
|  | (0.017) | (0.015) | (0.020) | (0.016) |
| Constant | 0.644*** | 0.366*** | 0.635*** | 0.371*** |
|  | (0.047) | (0.055) | (0.048) | (0.055) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.667 | 0.667 | 0.674 | 0.672 |
| Adj. R² | 0.633 | 0.632 | 0.638 | 0.635 |

| Panel B: Poverty Headcount $4.20 | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Headcount Ratio | -0.004* | -0.003 | -0.004** | -0.003* |
|  | (0.002) | (0.002) | (0.002) | (0.002) |
| Poverty Headcount Ratio Squared | 0.000 | 0.000** | 0.000 | 0.000** |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.051 | 0.048 |  |  |
|  | (0.032) | (0.033) |  |  |
| Governance Effectiveness |  |  | 0.048** | 0.022 |
|  |  |  | (0.024) | (0.022) |
| Building Quality Control Index |  |  | 0.007 | 0.010* |
|  |  |  | (0.006) | (0.005) |
| Urban Population % | 0.000 | 0.002* | 0.001 | 0.002** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.065*** | 0.044** | 0.058*** | 0.037* |
|  | (0.018) | (0.020) | (0.018) | (0.021) |
| Log Storm count | 0.006 | -0.007 | 0.006 | -0.005 |
|  | (0.016) | (0.014) | (0.016) | (0.014) |
| Log Flood count | -0.030* | -0.015 | -0.029 | -0.020 |
|  | (0.018) | (0.017) | (0.020) | (0.018) |
| Constant | 0.557*** | 0.224*** | 0.552*** | 0.226*** |
|  | (0.039) | (0.048) | (0.039) | (0.050) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.668 | 0.629 | 0.677 | 0.634 |
| Adj. R² | 0.636 | 0.593 | 0.644 | 0.596 |

| Panel C: Poverty Gap $3.00 | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap $3.00 | -0.007 | 0.000 | -0.009* | -0.001 |
|  | (0.005) | (0.005) | (0.005) | (0.005) |
| Poverty Gap $3.00 Squared | 0.000** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.077*** | 0.070** |  |  |
|  | (0.027) | (0.030) |  |  |
| Governance Effectiveness |  |  | 0.059*** | 0.035 |
|  |  |  | (0.022) | (0.021) |
| Building Quality Control Index |  |  | 0.008 | 0.012** |
|  |  |  | (0.006) | (0.005) |
| Urban Population % | 0.000 | 0.002* | 0.001 | 0.003*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.063*** | 0.043** | 0.056*** | 0.035* |
|  | (0.018) | (0.020) | (0.018) | (0.021) |
| Log Storm count | 0.006 | -0.005 | 0.007 | -0.003 |
|  | (0.016) | (0.014) | (0.015) | (0.014) |
| Log Flood count | -0.033* | -0.018 | -0.032 | -0.024 |
|  | (0.017) | (0.016) | (0.020) | (0.018) |
| Constant | 0.566*** | 0.258*** | 0.549*** | 0.254*** |
|  | (0.042) | (0.054) | (0.043) | (0.056) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.665 | 0.618 | 0.670 | 0.621 |
| Adj. R² | 0.633 | 0.581 | 0.636 | 0.581 |

| Panel D: Poverty Gap $8.30 | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap $8.30 | -0.005** | -0.005** | -0.005** | -0.005** |
|  | (0.002) | (0.002) | (0.002) | (0.002) |
| Poverty Gap $8.30 Squared | 0.000** | 0.000*** | 0.000* | 0.000*** |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.034 | 0.027 |  |  |
|  | (0.034) | (0.035) |  |  |
| Governance Effectiveness |  |  | 0.038 | 0.010 |
|  |  |  | (0.025) | (0.022) |
| Building Quality Control Index |  |  | 0.006 | 0.009 |
|  |  |  | (0.006) | (0.005) |
| Urban Population % | 0.000 | 0.002* | 0.000 | 0.002** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.067*** | 0.047** | 0.062*** | 0.041* |
|  | (0.018) | (0.020) | (0.018) | (0.021) |
| Log Storm count | 0.006 | -0.007 | 0.006 | -0.006 |
|  | (0.016) | (0.014) | (0.016) | (0.014) |
| Log Flood count | -0.029* | -0.014 | -0.029 | -0.019 |
|  | (0.017) | (0.016) | (0.019) | (0.017) |
| Constant | 0.554*** | 0.215*** | 0.553*** | 0.223*** |
|  | (0.038) | (0.047) | (0.039) | (0.048) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.675 | 0.642 | 0.683 | 0.648 |
| Adj. R² | 0.644 | 0.607 | 0.650 | 0.611 |

### Table A-3: Robustness Checks - Sample Restrictions

Notes: Panel A checks exclude small states (population < 1.5 million). Panel B excludes observations with disaster deaths in the top 1%. Panel C winsorizes disaster variables at the 95th percentile. All the models include regional fixed effects (coefficients not shown). Robust standard errors in parentheses. ***p<0.01, **p<0.05, *p<0.10.

| Panel A: Exclude Small States | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.004 | 0.001 | -0.004 | 0.001 |
|  | (0.003) | (0.003) | (0.003) | (0.003) |
| Poverty Gap Squared | 0.000* | -0.000 | 0.000 | -0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.095*** | 0.055 |  |  |
|  | (0.033) | (0.035) |  |  |
| Governance Effectiveness |  |  | 0.040* | 0.030 |
|  |  |  | (0.021) | (0.021) |
| Building Quality Control Index |  |  | 0.013** | 0.007 |
|  |  |  | (0.006) | (0.004) |
| Urban Population % | 0.000 | 0.002 | 0.002 | 0.003*** |
|  | (0.002) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.067*** | 0.028 | 0.050*** | 0.018 |
|  | (0.019) | (0.022) | (0.019) | (0.023) |
| Log Storm count | 0.005 | -0.000 | 0.008 | 0.000 |
|  | (0.014) | (0.014) | (0.014) | (0.015) |
| Log Flood count | -0.036* | -0.027 | -0.029 | -0.021 |
|  | (0.021) | (0.020) | (0.022) | (0.020) |
| Constant | 0.591*** | 0.380*** | 0.574*** | 0.369*** |
|  | (0.054) | (0.072) | (0.049) | (0.072) |
| N | 125 | 125 | 125 | 125 |
| R² | 0.754 | 0.672 | 0.753 | 0.674 |
| Adj. R² | 0.725 | 0.634 | 0.721 | 0.633 |

| Panel B: Exclude Top 1% Disasters | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.006* | -0.001 | -0.007** | -0.002 |
|  | (0.004) | (0.004) | (0.004) | (0.004) |
| Poverty Gap Squared | 0.000** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.067** | 0.064** |  |  |
|  | (0.029) | (0.031) |  |  |
| Governance Effectiveness |  |  | 0.053** | 0.031 |
|  |  |  | (0.023) | (0.021) |
| Building Quality Control Index |  |  | 0.007 | 0.011* |
|  |  |  | (0.006) | (0.005) |
| Urban Population % | 0.000 | 0.002* | 0.001 | 0.003*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.063*** | 0.035* | 0.057*** | 0.027 |
|  | (0.019) | (0.019) | (0.019) | (0.020) |
| Log Storm count | 0.006 | -0.008 | 0.006 | -0.007 |
|  | (0.017) | (0.014) | (0.016) | (0.014) |
| Log Flood count | -0.032* | -0.017 | -0.031 | -0.022 |
|  | (0.018) | (0.017) | (0.020) | (0.018) |
| Constant | 0.555*** | 0.240*** | 0.541*** | 0.237*** |
|  | (0.041) | (0.053) | (0.041) | (0.054) |
| N | 148 | 148 | 148 | 148 |
| R² | 0.663 | 0.620 | 0.669 | 0.623 |
| Adj. R² | 0.630 | 0.583 | 0.634 | 0.583 |

| Panel C: Winsorized Disasters (P95) | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.007* | -0.002 | -0.008** | -0.003 |
|  | (0.004) | (0.004) | (0.004) | (0.004) |
| Poverty Gap Squared | 0.000** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.067** | 0.065** |  |  |
|  | (0.028) | (0.031) |  |  |
| Governance Effectiveness |  |  | 0.053** | 0.031 |
|  |  |  | (0.023) | (0.021) |
| Building Quality Control Index |  |  | 0.008 | 0.011** |
|  |  |  | (0.006) | (0.005) |
| Urban Population % | 0.000 | 0.002* | 0.001 | 0.003*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.064*** | 0.044** | 0.056*** | 0.034 |
|  | (0.022) | (0.021) | (0.021) | (0.022) |
| Log Storm count | 0.006 | -0.006 | 0.006 | -0.003 |
|  | (0.018) | (0.015) | (0.017) | (0.015) |
| Log Flood count | -0.031* | -0.017 | -0.030 | -0.023 |
|  | (0.018) | (0.017) | (0.020) | (0.018) |
| Constant | 0.568*** | 0.253*** | 0.554*** | 0.248*** |
|  | (0.042) | (0.051) | (0.042) | (0.052) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.663 | 0.617 | 0.670 | 0.622 |
| Adj. R² | 0.630 | 0.581 | 0.636 | 0.583 |

### Table A-4: Robustness Checks - Alternative Disaster Measures

Notes: Panel A used the population normalized disaster count. Panel B analysis uses binary disaster indicators (any disasters vs. none) Panel C uses the Log disaster death, normalized with the population. All analyses include regional fixed effects (coefficients not shown). Robust standard errors in parentheses. ***p<0.01, **p<0.05, *p<0.10.

| Panel A: Disaster Count (Population Normalized) | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.006* | -0.003 | -0.006 | -0.004 |
|  | (0.004) | (0.003) | (0.004) | (0.003) |
| Poverty Gap Squared | 0.000** | 0.000 | 0.000* | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.064** | 0.054* |  |  |
|  | (0.029) | (0.029) |  |  |
| Governance Effectiveness |  |  | 0.049** | 0.041* |
|  |  |  | (0.022) | (0.022) |
| Building Quality Control Index |  |  | 0.013* | 0.002 |
|  |  |  | (0.007) | (0.005) |
| Urban Population % | 0.001 | 0.001 | 0.001 | 0.002* |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count (Normalized) | 0.056 | 0.080** | 0.051 | 0.080** |
|  | (0.048) | (0.036) | (0.050) | (0.036) |
| Log Storm count (Normalized) | 0.009 | -0.050** | 0.021 | -0.053** |
|  | (0.031) | (0.021) | (0.038) | (0.025) |
| Log Flood count (Normalized) | -0.005 | -0.050** | 0.009 | -0.048** |
|  | (0.031) | (0.021) | (0.030) | (0.022) |
| Constant | 0.575*** | 0.303*** | 0.554*** | 0.284*** |
|  | (0.050) | (0.049) | (0.044) | (0.048) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.649 | 0.648 | 0.664 | 0.648 |
| Adj. R² | 0.616 | 0.614 | 0.629 | 0.611 |

| Panel B: Binary Disaster Indicators | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.007** | -0.003 | -0.008** | -0.003 |
|  | (0.003) | (0.003) | (0.003) | (0.003) |
| Poverty Gap Squared | 0.000** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.065** | 0.056* |  |  |
|  | (0.028) | (0.031) |  |  |
| Governance Effectiveness |  |  | 0.057** | 0.028 |
|  |  |  | (0.022) | (0.021) |
| Building Quality Control Index |  |  | 0.008 | 0.011** |
|  |  |  | (0.005) | (0.005) |
| Urban Population % | 0.000 | 0.002** | 0.001 | 0.002*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Earthquake (binary) | 0.029 | 0.030 | 0.025 | 0.022 |
|  | (0.036) | (0.029) | (0.036) | (0.030) |
| Storm count (binary) | 0.010 | -0.024 | 0.004 | -0.030 |
|  | (0.051) | (0.036) | (0.050) | (0.037) |
| Flood count (binary) | -0.041 | -0.078 | -0.032 | -0.088 |
|  | (0.077) | (0.069) | (0.083) | (0.074) |
| Constant | 0.600*** | 0.338*** | 0.584*** | 0.353*** |
|  | (0.100) | (0.087) | (0.107) | (0.093) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.644 | 0.614 | 0.655 | 0.622 |
| Adj. R² | 0.610 | 0.577 | 0.619 | 0.583 |

| Panel C: Disaster Deaths (Population Normalized) | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.008** | -0.003 | -0.008** | -0.004 |
|  | (0.003) | (0.003) | (0.003) | (0.003) |
| Poverty Gap Squared | 0.000*** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.063** | 0.063** |  |  |
|  | (0.028) | (0.031) |  |  |
| Governance Effectiveness |  |  | 0.055** | 0.037* |
|  |  |  | (0.023) | (0.022) |
| Building Quality Control Index |  |  | 0.011** | 0.010** |
|  |  |  | (0.005) | (0.005) |
| Urban Population % | 0.001 | 0.002* | 0.001 | 0.002*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake Death (normalized) | 0.002 | 0.003 | -0.000 | 0.001 |
|  | (0.010) | (0.008) | (0.009) | (0.009) |
| Log Storm Death (normalized) | 0.031** | 0.005 | 0.034*** | 0.008 |
|  | (0.013) | (0.010) | (0.012) | (0.010) |
| Log Flood Death (normalized) | -0.007 | 0.010 | -0.010 | 0.006 |
|  | (0.015) | (0.015) | (0.016) | (0.016) |
| Constant | 0.545*** | 0.248*** | 0.533*** | 0.237*** |
|  | (0.044) | (0.055) | (0.041) | (0.056) |
| N | 150 | 150 | 150 | 150 |
| R² | 0.659 | 0.608 | 0.675 | 0.614 |
| Adj. R² | 0.626 | 0.570 | 0.641 | 0.574 |

### Table A-5: Robustness Checks - Alternative Dependent Variable

Notes: Panel A and Panel B use MPI Housing Deprivation (reversed, so higher = better) and unit-weighted proportion as the dependent variable, respectively. The models include regional fixed effects (coefficients not shown). Robust standard errors in parentheses. ***p<0.01, **p<0.05, *p<0.10.

| Panel A: MPI Housing Deprivation | GDP Model | Institutional Model |
| --- | --- | --- |
| Poverty Gap | 0.061 | -0.397 |
|  | (0.313) | (0.318) |
| Poverty Gap Squared | -0.006 | -0.003 |
|  | (0.005) | (0.006) |
| Log GDP PC | 14.577*** |  |
|  | (2.580) |  |
| Governance Effectiveness |  | 8.639*** |
|  |  | (2.305) |
| Building Quality Control Index |  | -0.455 |
|  |  | (0.548) |
| Urban Population % | 0.131 | 0.289*** |
|  | (0.094) | (0.102) |
| Log Earthquake count | 3.565** | 3.190* |
|  | (1.573) | (1.754) |
| Log Storm count | 2.011* | 1.927 |
|  | (1.094) | (1.287) |
| Log Flood count | -5.427*** | -4.009** |
|  | (1.520) | (1.961) |
| Constant | 95.396*** | 89.930*** |
|  | (3.811) | (3.819) |
| Observations | 92 | 92 |
| R-squared | 0.794 | 0.738 |
| Adj. R-squared | 0.762 | 0.694 |
| Region Fixed Effects | Yes | Yes |

| Panel B: Unit-weighted proportion | VULN (GDP) | ONLY (GDP) | VULN (Inst.) | ONLY (Inst.) |
| --- | --- | --- | --- | --- |
| Poverty Gap | -0.006* | -0.001 | -0.007* | -0.002 |
|  | (0.004) | (0.003) | (0.004) | (0.003) |
| Poverty Gap Squared | 0.000** | 0.000 | 0.000** | 0.000 |
|  | (0.000) | (0.000) | (0.000) | (0.000) |
| Log GDP PC | 0.070** | 0.053* |  |  |
|  | (0.031) | (0.030) |  |  |
| Governance Effectiveness |  |  | 0.058** | 0.037 |
|  |  |  | (0.027) | (0.023) |
| Building Quality Control Index |  |  | 0.008 | 0.005 |
|  |  |  | (0.007) | (0.005) |
| Urban Population % | 0.000 | 0.002** | 0.001 | 0.002*** |
|  | (0.001) | (0.001) | (0.001) | (0.001) |
| Log Earthquake count | 0.043* | 0.035* | 0.036 | 0.030 |
|  | (0.024) | (0.018) | (0.025) | (0.019) |
| Log Storm count | 0.022 | 0.008 | 0.022 | 0.008 |
|  | (0.019) | (0.013) | (0.019) | (0.013) |
| Log Flood count | -0.032* | -0.024* | -0.031 | -0.024 |
|  | (0.018) | (0.014) | (0.021) | (0.016) |
| Constant | 0.395*** | 0.163*** | 0.380*** | 0.152*** |
|  | (0.047) | (0.038) | (0.049) | (0.040) |
| Observations | 150 | 150 | 150 | 150 |
| R-squared | 0.565 | 0.668 | 0.574 | 0.670 |
| Adj. R-squared | 0.523 | 0.636 | 0.529 | 0.636 |

### Table A-6: Regional Heterogeneity (Jackknife Analysis)

Notes: Each row presents results from models that exclude one World Bank region. This tests whether results are driven by any single region. All models include remaining regional fixed effects. Robust standard errors in parentheses. ***p<0.01, **p<0.05, *p<0.10.

| Excluded Region | N | Model Type | Poverty (Gap) | Earthquake (Log) | Storm (Log) | Flood (Log) | R² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-Saharan Africa | 109 | (1) GDP / Vuln | -0.012** | 0.082*** | 0.012 | -0.054** | 0.387 |
|  |  | (2) GDP / Only | -0.011* | 0.061*** | 0.001 | -0.031 | 0.579 |
|  |  | (3) Gov / Vuln | -0.013** | 0.074*** | 0.009 | -0.042* | 0.401 |
|  |  | (4) Gov / Only | -0.013** | 0.051** | 0.001 | -0.031 | 0.579 |
| Europe & Central Asia | 106 | (1) GDP / Vuln | -0.004 | 0.056*** | 0.028 | -0.043** | 0.752 |
|  |  | (2) GDP / Only | -0.002 | 0.024 | 0.004 | 0.006 | 0.576 |
|  |  | (3) Gov / Vuln | -0.004 | 0.050** | 0.026 | -0.046** | 0.772 |
|  |  | (4) Gov / Only | -0.003 | 0.018 | 0.002 | -0.007 | 0.585 |
| Middle East & N. Africa | 137 | (1) GDP / Vuln | -0.007* | 0.067*** | 0.003 | -0.024 | 0.676 |
|  |  | (2) GDP / Only | 0.000 | 0.046** | -0.009 | -0.012 | 0.620 |
|  |  | (3) Gov / Vuln | -0.008** | 0.058*** | 0.003 | -0.019 | 0.683 |
|  |  | (4) Gov / Only | -0.001 | 0.035 | -0.004 | -0.018 | 0.618 |
| Latin America & Carib. | 126 | (1) GDP / Vuln | -0.006* | 0.061*** | -0.018 | -0.015 | 0.697 |
|  |  | (2) GDP / Only | -0.003 | 0.054** | -0.017 | -0.021 | 0.606 |
|  |  | (3) Gov / Vuln | -0.007** | 0.056*** | -0.017 | -0.016 | 0.701 |
|  |  | (4) Gov / Only | -0.004 | 0.046* | -0.013 | -0.029 | 0.616 |
| East Asia & Pacific | 130 | (1) GDP / Vuln | -0.005 | 0.050** | 0.004 | -0.036* | 0.671 |
|  |  | (2) GDP / Only | -0.000 | 0.029 | -0.004 | -0.030* | 0.665 |
|  |  | (3) Gov / Vuln | -0.007* | 0.045** | 0.007 | -0.039* | 0.673 |
|  |  | (4) Gov / Only | -0.001 | 0.027 | -0.004 | -0.029 | 0.672 |
| South Asia | 144 | (1) GDP / Vuln | -0.007* | 0.059*** | 0.006 | -0.026 | 0.675 |
|  |  | (2) GDP / Only | -0.002 | 0.039** | -0.005 | -0.011 | 0.638 |
|  |  | (3) Gov / Vuln | -0.008** | 0.055*** | 0.006 | -0.025 | 0.679 |
|  |  | (4) Gov / Only | -0.003 | 0.034 | -0.003 | -0.017 | 0.639 |
| North America | 148 | (1) GDP / Vuln | -0.007* | 0.065*** | 0.007 | -0.032* | 0.662 |
|  |  | (2) GDP / Only | -0.002 | 0.043** | -0.005 | -0.017 | 0.615 |
|  |  | (3) Gov / Vuln | -0.008** | 0.059*** | 0.007 | -0.031 | 0.668 |
|  |  | (4) Gov / Only | -0.003 | 0.035 | -0.003 | -0.022 | 0.618 |

# Appendix C. Full Regression Results

### Table A-7: Determinants of Non-Fragile Housing (Stage 1)

Panel A: Baseline Models

| Variable | M1.Base | M2.Poverty | M3.Income | M4.Urb | M5.Gov | M6.Full |
| --- | --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0050*** | -0.0134*** | -0.0075** | -0.0074** | -0.0076** | -0.0072** |
|  | (0.0017) | (0.0027) | (0.0032) | (0.0033) | (0.0032) | (0.0032) |
| Poverty Gap Squared |  | 0.0003*** | 0.0002*** | 0.0002*** | 0.0002*** | 0.0002** |
|  |  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  |  | 0.0700*** | 0.0679** | 0.0305 | 0.0210 |
|  |  |  | (0.0239) | (0.0280) | (0.0324) | (0.0339) |
| Urban Population % |  |  |  | 0.0002 | 0.0002 | 0.0002 |
|  |  |  |  | (0.0012) | (0.0011) | (0.0012) |
| Government Effectiveness |  |  |  |  | 0.0473* | 0.0484* |
|  |  |  |  |  | (0.0263) | (0.0266) |
| Building Quality Index |  |  |  |  |  | 0.0080 |
|  |  |  |  |  |  | (0.0053) |
| Region FE: Middle East & North Africa | 0.0927 | 0.0644 | 0.0536 | 0.0506 | 0.0839 | 0.0620 |
|  | (0.0771) | (0.0722) | (0.0679) | (0.0716) | (0.0728) | (0.0734) |
| Region FE: Latin America & Caribbean | 0.1061* | 0.0881 | 0.0743 | 0.0729 | 0.1080** | 0.1128** |
|  | (0.0618) | (0.0586) | (0.0572) | (0.0593) | (0.0546) | (0.0537) |
| Region FE: Europe & Central Asia | -0.0031 | -0.0455 | -0.1027** | -0.1025** | -0.0786 | -0.0849* |
|  | (0.0525) | (0.0511) | (0.0511) | (0.0514) | (0.0521) | (0.0505) |
| Region FE: North America | 0.2444*** | 0.2009*** | 0.1059* | 0.1052 | 0.1135* | 0.0982* |
|  | (0.0540) | (0.0508) | (0.0641) | (0.0642) | (0.0580) | (0.0552) |
| Region FE: Sub-Saharan Africa | -0.3124*** | -0.2552*** | -0.2682*** | -0.2690*** | -0.2558*** | -0.2672*** |
|  | (0.0572) | (0.0547) | (0.0552) | (0.0556) | (0.0530) | (0.0515) |
| Region FE: South Asia | -0.2547** | -0.2628** | -0.2375** | -0.2358** | -0.2271** | -0.2417** |
|  | (0.1120) | (0.1076) | (0.1025) | (0.1036) | (0.0984) | (0.0951) |
| Constant | 0.5906*** | 0.5512*** | 0.5879*** | 0.5886*** | 0.5702*** | 0.5793*** |
|  | (0.0442) | (0.0402) | (0.0416) | (0.0419) | (0.0401) | (0.0395) |
| Observations | 150 | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.5913 | 0.6190 | 0.6409 | 0.6410 | 0.6484 | 0.6539 |
| Adj. R-squared | 0.5711 | 0.5974 | 0.6178 | 0.6151 | 0.6204 | 0.6236 |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel B: Disaster Learning Models (Model A)

| Variable | D1.Pov | D2.Econ | D3.Urb | D4.Inst | D5.Policy |
| --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0128*** | -0.0068** | -0.0067* | -0.0071** | -0.0065* |
|  | (0.0028) | (0.0035) | (0.0035) | (0.0034) | (0.0036) |
| Poverty Gap Squared | 0.0002*** | 0.0002** | 0.0002** | 0.0001** | 0.0001** |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  | 0.0716*** | 0.0668** | 0.0334 | 0.0280 |
|  |  | (0.0249) | (0.0287) | (0.0354) | (0.0360) |
| Urban Population % |  |  | 0.0004 | 0.0004 | 0.0005 |
|  |  |  | (0.0012) | (0.0012) | (0.0012) |
| Government Effectiveness |  |  |  | 0.0431 | 0.0403 |
|  |  |  |  | (0.0284) | (0.0282) |
| Building Quality Index |  |  |  |  | 0.0065 |
|  |  |  |  |  | (0.0064) |
| Earthquake Count (Log) | 0.0580*** | 0.0626*** | 0.0632*** | 0.0616*** | 0.0579*** |
|  | (0.0188) | (0.0180) | (0.0181) | (0.0179) | (0.0180) |
| Storm Count (Log) | 0.0144 | 0.0053 | 0.0050 | 0.0036 | 0.0043 |
|  | (0.0164) | (0.0163) | (0.0164) | (0.0158) | (0.0158) |
| Flood Count (Log) | -0.0298* | -0.0307* | -0.0315* | -0.0254 | -0.0312 |
|  | (0.0176) | (0.0171) | (0.0175) | (0.0182) | (0.0201) |
| Constant | 0.5148*** | 0.5572*** | 0.5592*** | 0.5426*** | 0.5548*** |
|  | (0.0382) | (0.0405) | (0.0406) | (0.0402) | (0.0435) |
| Observations | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.6449 | 0.6669 | 0.6673 | 0.6730 | 0.6758 |
| Adj. R-squared | 0.6166 | 0.6378 | 0.6355 | 0.6391 | 0.6395 |
| Regional FE | Yes | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel C: Heterogeneous Effects (Interaction Models: Model A)

| Variable | Int.Pov | Int.GDP | Int.WGI | Int.BQCI |
| --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0071** | -0.0065* | -0.0073** | -0.0056 |
|  | (0.0033) | (0.0033) | (0.0033) | (0.0035) |
| Poverty Gap Squared | 0.0001 | 0.0001** | 0.0002** | 0.0001** |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) | 0.0651** | 0.0763*** | 0.0349 | 0.0633** |
|  | (0.0293) | (0.0284) | (0.0357) | (0.0296) |
| Urban Population % | 0.0003 | 0.0002 | 0.0003 | 0.0008 |
|  | (0.0012) | (0.0012) | (0.0011) | (0.0012) |
| Government Effectiveness |  |  | 0.0518* |  |
|  |  |  | (0.0285) |  |
| Building Quality Index |  |  |  | 0.0101 |
|  |  |  |  | (0.0064) |
| Earthquake Count (Log) | 0.0468*** | 0.0538*** | 0.0557*** | 0.0434* |
|  | (0.0170) | (0.0181) | (0.0182) | (0.0253) |
| Storm Count (Log) | 0.0106 | 0.0058 | 0.0042 | 0.0043 |
|  | (0.0167) | (0.0166) | (0.0159) | (0.0172) |
| Flood Count (Log) | -0.0183 | -0.0260 | -0.0206 | -0.0330* |
|  | (0.0157) | (0.0171) | (0.0185) | (0.0196) |
| Poverty × Earthquake | -0.0041*** |  |  |  |
|  | (0.0012) |  |  |  |
| Poverty × Storm | 0.0012* |  |  |  |
|  | (0.0007) |  |  |  |
| Poverty × Flood | 0.0033*** |  |  |  |
|  | (0.0012) |  |  |  |
| GDP × Earthquake |  | 0.0352** |  |  |
|  |  | (0.0169) |  |  |
| GDP × Storm |  | -0.0238** |  |  |
|  |  | (0.0102) |  |  |
| GDP × Flood |  | 0.0009 |  |  |
|  |  | (0.0163) |  |  |
| Gov. Effectiveness × Earthquake |  |  | 0.0349** |  |
|  |  |  | (0.0172) |  |
| Gov. Effectiveness × Storm |  |  | -0.0312** |  |
|  |  |  | (0.0147) |  |
| Gov. Effectiveness × Flood |  |  | 0.0054 |  |
|  |  |  | (0.0172) |  |
| Building Quality × Earthquake |  |  |  | 0.0031 |
|  |  |  |  | (0.0076) |
| Building Quality × Storm |  |  |  | -0.0063 |
|  |  |  |  | (0.0050) |
| Building Quality × Flood |  |  |  | 0.0087 |
|  |  |  |  | (0.0058) |
| Constant | 0.5713*** | 0.5688*** | 0.5484*** | 0.5545*** |
|  | (0.0403) | (0.0413) | (0.0395) | (0.0441) |
| Observations | 150 | 150 | 150 | 150 |
| R-squared | 0.6797 | 0.6794 | 0.6866 | 0.6821 |
| Adj. R-squared | 0.6411 | 0.6408 | 0.6462 | 0.6412 |
| Regional FE | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel D: Disaster Learning Models (Model B)

| Variable | VULN-M.D1.Pov | VULN-M.D2.Econ | VULN-M.D3.Urb | VULN-M.D4.Inst | VULN-M.D5.Policy |
| --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0136*** | -0.0077** | -0.0077** | -0.0080*** | -0.0073** |
|  | (0.0026) | (0.0032) | (0.0032) | (0.0031) | (0.0032) |
| Poverty Gap Squared | 0.0002*** | 0.0002*** | 0.0002*** | 0.0002*** | 0.0002** |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  | 0.0689*** | 0.0676** | 0.0265 | 0.0204 |
|  |  | (0.0243) | (0.0283) | (0.0344) | (0.0347) |
| Urban Population % |  |  | 0.0001 | 0.0001 | 0.0002 |
|  |  |  | (0.0012) | (0.0012) | (0.0012) |
| Government Effectiveness |  |  |  | 0.0517* | 0.0498* |
|  |  |  |  | (0.0287) | (0.0288) |
| Building Quality Index |  |  |  |  | 0.0073 |
|  |  |  |  |  | (0.0061) |
| 3 Hazards Combined (Log) | 0.0116 | 0.0081 | 0.0079 | 0.0129 | 0.0042 |
|  | (0.0194) | (0.0193) | (0.0194) | (0.0198) | (0.0223) |
| Constant | 0.5451*** | 0.5831*** | 0.5837*** | 0.5604*** | 0.5754*** |
|  | (0.0385) | (0.0418) | (0.0423) | (0.0412) | (0.0451) |
| Observations | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.6206 | 0.6417 | 0.6417 | 0.6502 | 0.6541 |
| Adj. R-squared | 0.5962 | 0.6159 | 0.6131 | 0.6196 | 0.6210 |
| Regional FE | Yes | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel E: Heterogeneous Effects (Interaction Models: Model B)

| Variable | VULN-M.Int.Pov | VULN-M.Int.GDP | VULN-M.Int.WGI | VULN-M.Int.BQCI |
| --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0073** | -0.0081*** | -0.0084*** | -0.0058* |
|  | (0.0031) | (0.0031) | (0.0030) | (0.0034) |
| Poverty Gap Squared | 0.0001** | 0.0002*** | 0.0002*** | 0.0002** |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) | 0.0665** | 0.0610** | 0.0250 | 0.0635** |
|  | (0.0274) | (0.0283) | (0.0346) | (0.0294) |
| Urban Population % | 0.0002 | 0.0002 | 0.0001 | 0.0001 |
|  | (0.0012) | (0.0012) | (0.0012) | (0.0012) |
| Government Effectiveness |  |  | 0.0488* |  |
|  |  |  | (0.0270) |  |
| Building Quality Index |  |  |  | 0.0103* |
|  |  |  |  | (0.0058) |
| 3 Hazards Combined (Log) | 0.0195 | 0.0127 | 0.0167 | -0.0090 |
|  | (0.0176) | (0.0199) | (0.0219) | (0.0237) |
| Poverty × 3 Hazards | 0.0026** |  |  |  |
|  | (0.0012) |  |  |  |
| GDP × 3 Hazards |  | -0.0117 |  |  |
|  |  | (0.0165) |  |  |
| Governance Effectiveness × 3 Hazards |  |  | -0.0111 |  |
|  |  |  | (0.0204) |  |
| Building Quality × 3 Hazards |  |  |  | 0.0080 |
|  |  |  |  | (0.0064) |
| Constant | 0.5965*** | 0.5882*** | 0.5647*** | 0.5741*** |
|  | (0.0428) | (0.0440) | (0.0416) | (0.0492) |
| Observations | 150 | 150 | 150 | 150 |
| R-squared | 0.6468 | 0.6432 | 0.6516 | 0.6531 |
| Adj. R-squared | 0.6159 | 0.6120 | 0.6182 | 0.6200 |
| Regional FE | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

### Table A-8: Determinants of Robust-Only Index (Stage 2)

Panel A: Baseline Models

| Variable | M1.Base | M2.Poverty | M3.Income | M4.Urb | M5.Gov | M6.Full |
| --- | --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0038*** | -0.0100*** | -0.0028 | -0.0024 | -0.0025 | -0.0019 |
|  | (0.0013) | (0.0031) | (0.0035) | (0.0034) | (0.0034) | (0.0033) |
| Poverty Gap Squared |  | 0.0002*** | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
|  |  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  |  | 0.0851*** | 0.0628** | 0.0506 | 0.0393 |
|  |  |  | (0.0255) | (0.0302) | (0.0318) | (0.0301) |
| Urban Population % |  |  |  | 0.0018* | 0.0018* | 0.0018* |
|  |  |  |  | (0.0010) | (0.0010) | (0.0010) |
| Government Effectiveness |  |  |  |  | 0.0154 | 0.0167 |
|  |  |  |  |  | (0.0214) | (0.0210) |
| Building Quality Index |  |  |  |  |  | 0.0094** |
|  |  |  |  |  |  | (0.0044) |
| Region FE: Middle East & North Africa | 0.2753*** | 0.2544*** | 0.2413*** | 0.2082** | 0.2191** | 0.1933** |
|  | (0.0919) | (0.0899) | (0.0874) | (0.0878) | (0.0912) | (0.0910) |
| Region FE: Latin America & Caribbean | -0.1367** | -0.1500** | -0.1668*** | -0.1829*** | -0.1714*** | -0.1657*** |
|  | (0.0587) | (0.0590) | (0.0554) | (0.0581) | (0.0605) | (0.0574) |
| Region FE: Europe & Central Asia | 0.2094*** | 0.1781*** | 0.1085 | 0.1105 | 0.1183* | 0.1110 |
|  | (0.0630) | (0.0657) | (0.0675) | (0.0675) | (0.0686) | (0.0679) |
| Region FE: North America | -0.2238*** | -0.2559*** | -0.3716*** | -0.3793*** | -0.3766*** | -0.3946*** |
|  | (0.0571) | (0.0607) | (0.0734) | (0.0739) | (0.0743) | (0.0765) |
| Region FE: Sub-Saharan Africa | -0.0672 | -0.0249 | -0.0408 | -0.0495 | -0.0453 | -0.0587 |
|  | (0.0546) | (0.0542) | (0.0543) | (0.0538) | (0.0545) | (0.0526) |
| Region FE: South Asia | 0.0815 | 0.0755 | 0.1063 | 0.1257 | 0.1285 | 0.1113 |
|  | (0.1161) | (0.1120) | (0.1050) | (0.1075) | (0.1073) | (0.1046) |
| Constant | 0.2355*** | 0.2064*** | 0.2511*** | 0.2592*** | 0.2532*** | 0.2639*** |
|  | (0.0537) | (0.0478) | (0.0492) | (0.0503) | (0.0512) | (0.0508) |
| Observations | 150 | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.5308 | 0.5513 | 0.5953 | 0.6048 | 0.6059 | 0.6163 |
| Adj. R-squared | 0.5077 | 0.5259 | 0.5693 | 0.5764 | 0.5745 | 0.5827 |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel B: Disaster Learning Models (Model A)

| Variable | D1.Pov | D2.Econ | D3.Urb | D4.Inst | D5.Policy |
| --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0098*** | -0.0025 | -0.0020 | -0.0021 | -0.0012 |
|  | (0.0032) | (0.0036) | (0.0036) | (0.0036) | (0.0037) |
| Poverty Gap Squared | 0.0002** | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  | 0.0880*** | 0.0644** | 0.0539* | 0.0459 |
|  |  | (0.0269) | (0.0311) | (0.0316) | (0.0308) |
| Urban Population % |  |  | 0.0019* | 0.0019* | 0.0021** |
|  |  |  | (0.0010) | (0.0010) | (0.0010) |
| Government Effectiveness |  |  |  | 0.0135 | 0.0094 |
|  |  |  |  | (0.0214) | (0.0205) |
| Building Quality Index |  |  |  |  | 0.0096* |
|  |  |  |  |  | (0.0054) |
| Earthquake Count (Log) | 0.0343* | 0.0400** | 0.0428** | 0.0423** | 0.0369* |
|  | (0.0208) | (0.0204) | (0.0199) | (0.0200) | (0.0211) |
| Storm Count (Log) | 0.0070 | -0.0041 | -0.0056 | -0.0061 | -0.0051 |
|  | (0.0146) | (0.0143) | (0.0143) | (0.0144) | (0.0144) |
| Flood Count (Log) | -0.0122 | -0.0133 | -0.0169 | -0.0150 | -0.0235 |
|  | (0.0176) | (0.0168) | (0.0166) | (0.0168) | (0.0178) |
| Constant | 0.1848*** | 0.2369*** | 0.2466*** | 0.2414*** | 0.2594*** |
|  | (0.0483) | (0.0502) | (0.0515) | (0.0518) | (0.0531) |
| Observations | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.5635 | 0.6086 | 0.6198 | 0.6205 | 0.6288 |
| Adj. R-squared | 0.5287 | 0.5743 | 0.5834 | 0.5812 | 0.5873 |
| Regional FE | Yes | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel C: Heterogeneous Effects (Interaction Models: Model A)

| Variable | Int.Pov | Int.GDP | Int.WGI | Int.BQCI |
| --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0022 | -0.0022 | -0.0027 | -0.0013 |
|  | (0.0035) | (0.0034) | (0.0033) | (0.0036) |
| Poverty Gap Squared | 0.0000 | 0.0001 | 0.0001 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) | 0.0621* | 0.0639** | 0.0543* | 0.0549* |
|  | (0.0331) | (0.0303) | (0.0311) | (0.0306) |
| Urban Population % | 0.0019* | 0.0016* | 0.0017* | 0.0019* |
|  | (0.0010) | (0.0009) | (0.0010) | (0.0011) |
| Government Effectiveness |  |  | 0.0171 |  |
|  |  |  | (0.0220) |  |
| Building Quality Index |  |  |  | 0.0098* |
|  |  |  |  | (0.0055) |
| Earthquake Count (Log) | 0.0315* | 0.0292* | 0.0346* | 0.0318 |
|  | (0.0179) | (0.0170) | (0.0182) | (0.0232) |
| Storm Count (Log) | -0.0007 | -0.0008 | -0.0026 | -0.0038 |
|  | (0.0147) | (0.0146) | (0.0136) | (0.0143) |
| Flood Count (Log) | -0.0059 | -0.0058 | -0.0061 | -0.0262 |
|  | (0.0152) | (0.0158) | (0.0156) | (0.0192) |
| Poverty × Earthquake | -0.0028** |  |  |  |
|  | (0.0013) |  |  |  |
| Poverty × Storm | 0.0008 |  |  |  |
|  | (0.0007) |  |  |  |
| Poverty × Flood | 0.0028** |  |  |  |
|  | (0.0012) |  |  |  |
| GDP × Earthquake |  | 0.0448*** |  |  |
|  |  | (0.0167) |  |  |
| GDP × Storm |  | -0.0173 |  |  |
|  |  | (0.0110) |  |  |
| GDP × Flood |  | -0.0234 |  |  |
|  |  | (0.0156) |  |  |
| Gov. Effectiveness × Earthquake |  |  | 0.0447** |  |
|  |  |  | (0.0185) |  |
| Gov. Effectiveness × Storm |  |  | -0.0314** |  |
|  |  |  | (0.0135) |  |
| Gov. Effectiveness × Flood |  |  | -0.0119 |  |
|  |  |  | (0.0150) |  |
| Building Quality × Earthquake |  |  |  | 0.0073 |
|  |  |  |  | (0.0080) |
| Building Quality × Storm |  |  |  | -0.0027 |
|  |  |  |  | (0.0045) |
| Building Quality × Flood |  |  |  | -0.0044 |
|  |  |  |  | (0.0057) |
| Constant | 0.2568*** | 0.2646*** | 0.2477*** | 0.2712*** |
|  | (0.0534) | (0.0510) | (0.0499) | (0.0562) |
| Observations | 150 | 150 | 150 | 150 |
| R-squared | 0.6296 | 0.6400 | 0.6439 | 0.6330 |
| Adj. R-squared | 0.5851 | 0.5966 | 0.5981 | 0.5857 |
| Regional FE | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel D: Disaster Learning Models (Model B)

| Variable | D1.Pov | D2.Econ | D3.Urb | D4.Inst | D5.Policy |
| --- | --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0102*** | -0.0029 | -0.0024 | -0.0026 | -0.0015 |
|  | (0.0031) | (0.0035) | (0.0035) | (0.0035) | (0.0036) |
| Poverty Gap Squared | 0.0002*** | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) |  | 0.0846*** | 0.0627** | 0.0497 | 0.0406 |
|  |  | (0.0259) | (0.0303) | (0.0312) | (0.0304) |
| Urban Population % |  |  | 0.0017* | 0.0017* | 0.0019* |
|  |  |  | (0.0010) | (0.0010) | (0.0010) |
| Government Effectiveness |  |  |  | 0.0164 | 0.0135 |
|  |  |  |  | (0.0204) | (0.0194) |
| Building Quality Index |  |  |  |  | 0.0110** |
|  |  |  |  |  | (0.0050) |
| 3 Hazards Combined (Log) | 0.0079 | 0.0036 | 0.0012 | 0.0027 | -0.0103 |
|  | (0.0183) | (0.0173) | (0.0178) | (0.0176) | (0.0192) |
| Constant | 0.2023*** | 0.2490*** | 0.2585*** | 0.2511*** | 0.2735*** |
|  | (0.0459) | (0.0482) | (0.0501) | (0.0503) | (0.0514) |
| Observations | 150 | 150 | 150 | 150 | 150 |
| R-squared | 0.5523 | 0.5955 | 0.6049 | 0.6060 | 0.6177 |
| Adj. R-squared | 0.5235 | 0.5664 | 0.5734 | 0.5715 | 0.5811 |
| Regional FE | Yes | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

Panel E: Heterogeneous Effects (Interaction Models: Model B)

| Variable | ONLY-M.Int.Pov | ONLY-M.Int.GDP | ONLY-M.Int.WGI | ONLY-M.Int.BQCI |
| --- | --- | --- | --- | --- |
| Poverty Gap ($4.20/day) | -0.0021 | -0.0033 | -0.0032 | -0.0016 |
|  | (0.0033) | (0.0033) | (0.0033) | (0.0035) |
| Poverty Gap Squared | 0.0000 | 0.0001 | 0.0001 | 0.0001 |
|  | (0.0001) | (0.0001) | (0.0001) | (0.0001) |
| Log GDP per Capita (PPP) | 0.0618** | 0.0498* | 0.0470 | 0.0505* |
|  | (0.0304) | (0.0298) | (0.0312) | (0.0288) |
| Urban Population % | 0.0018* | 0.0020* | 0.0018* | 0.0019* |
|  | (0.0010) | (0.0010) | (0.0010) | (0.0010) |
| Government Effectiveness (WGI) |  |  | 0.0114 |  |
|  |  |  | (0.0200) |  |
| Building Quality Index |  |  |  | 0.0107** |
|  |  |  |  | (0.0048) |
| 3 Hazards Combined (Log) | 0.0114 | 0.0106 | 0.0093 | -0.0104 |
|  | (0.0160) | (0.0183) | (0.0170) | (0.0184) |
| Poverty × 3 Hazards | 0.0023* |  |  |  |
|  | (0.0014) |  |  |  |
| GDP × 3 Hazards |  | -0.0228 |  |  |
|  |  | (0.0182) |  |  |
| Gov. Effectiveness × 3 Hazards |  |  | -0.0195 |  |
|  |  |  | (0.0176) |  |
| Building Quality × 3 Hazards |  |  |  | -0.0014 |
|  |  |  |  | (0.0054) |
| Constant | 0.2697*** | 0.2674*** | 0.2586*** | 0.2841*** |
|  | (0.0502) | (0.0497) | (0.0495) | (0.0548) |
| Observations | 150 | 150 | 150 | 150 |
| R-squared | 0.6102 | 0.6128 | 0.6115 | 0.6172 |
| Adj. R-squared | 0.5760 | 0.5789 | 0.5743 | 0.5806 |
| Regional FE | Yes | Yes | Yes | Yes |

Note: *** p<0.01, ** p<0.05, * p<0.10.

# Appendix D. GEM taxonomy mapping

Table A-9. GEM taxonomy mapping (representative cases)

| No | GEM Taxonomy | Description | Combined | Seismic | Flood/ Storm | Wind | Observation and Verification by this study |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GROUP 1 — Unreinforced Masonry (MUR): 31 entries ¦ All fragile except cement-mortar subtypes |  |  |  |  |  |  |  |
| 1 | MUR | Masonry, unreinforced (generic) | fragile | fragile | fragile | fragile | Masonry without a lateral load resistance system is considered fragile. Adobe, which is made of sun-dried clay, has near-zero tensile and shear strength. |
| 2 | MUR+ADO | Masonry, unreinforced + adobe blocks | fragile | fragile | fragile | fragile |  |
| 3 | MUR+STDRE+MOC | Masonry, unreinforced + dressed stone + cement mortar | median | median | fragile | median | Masonry made with cement mortar and dressed stone or fired clay solid bricks is meaningfully stronger than masonry made with lime or mud mortar. The seismic and wind performance could improve to the median level. However, flood resistance remains fragile because porous stone absorbs water and weakens the cement-stone bond. |
| 4 | MUR+CLBRS+MOC | Masonry, unreinforced + fired clay solid bricks + cement mortar | median | median | fragile | median |  |
| GROUP 2 — Earth Construction (EU, ER, E+): 5 entries ¦ All fragile |  |  |  |  |  |  |  |
| 5 | EU | Earth, unreinforced | fragile | fragile | fragile | fragile | Pure, unreinforced earth construction. It is the weakest load-bearing material for all hazards. Earth dissolves in water, so it is susceptible to flooding. |
| GROUP 3 — Confined Masonry (MCF): 11 entries ¦ Median/Robust (earthquake); Fragile (flood) |  |  |  |  |  |  |  |
| 6 | MCF | Masonry, confined (generic) | median | median | fragile | median | RC tie-columns and tie-beams provide confinement, box behavior and ductility. It will be better than MUR in earthquakes and wind. Flood remains fragile: masonry infill panels vulnerable to hydrostatic pressure and erosion regardless of RC frame. |
| 7 | MCF+CB | Masonry, confined + concrete block (unknown type) | robust | median | fragile | median | Confinement with an RC frame using concrete blocks or fired clay solid bricks provides high overall structural integrity, making earthquake and wind median. Combined tier is assigned as robust, despite the individual hazard tier being up to the median. The study follows the original classification. |
| 8 | MCF+CLBRS | Masonry, confined + fired clay solid bricks | robust | median | fragile | median |  |
| GROUP 4 — Reinforced Masonry (MR): 7 entries ¦ Median |  |  |  |  |  |  |  |
| 9 | MR | Masonry, reinforced (generic) | median | median | fragile | median | The internal vertical and horizontal reinforcement provides the tensile capacity that is absent in the MUR. Masonry units are still vulnerable to inundation. |
| GROUP 5 — Reinforced Concrete (CR): 18 entries ¦ Median/Robust (height-dependent) |  |  |  |  |  |  |  |
| 10 | CR+CIP [sys: LDUAL] [ht: default] | RC cast-in-place + dual frame-wall + MULTI-STOREY (>3 floors) | robust | robust | median | median | The multi-story RC dual system/moment frame is the globally recognized benchmark for robust structural systems. It is robust in both earthquakes and combined loads. Median flood resistance (upper floors unaffected). |
| 11 | CR+CIP [sys: LFM] [ht: default] | RC cast-in-place + moment frame + MULTI-STOREY | robust | robust | median | median |  |
| 12 | CR+CIP [sys: LDUAL] [ht: HBET:3,1] | RC cast-in-place + dual frame-wall + LOW RISE (1-3 floors) | median | robust | fragile | robust | The height creates a paradox: the best seismic system, yet the ground floor is vulnerable to flooding, which makes the combined index for low-rise buildings median. |
| 13 | CR+CIP [sys: LFINF] [ht: HBET:3,1] | RC cast-in-place + infilled frame + LOW RISE | median | median | fragile | robust | An infilled frame creates a soft-story risk, making the earthquake tier median. |
| GROUP 6 — Steel (S, SL, SR, SRC): 8 entries ¦ Robust (earthquake/combined); Median (flood/wind) |  |  |  |  |  |  |  |
| 14 | S | Steel (generic) | robust | robust | median | median | Steel structures offer high ductility and energy dissipation for all hazard. |
| 15 | SRC | Concrete composite with steel section (SRC) | robust | robust | median | median | SRC is the highest-performance structural systems. |
| 16 | S+S99+SC99 | Steel: CROSS-HAZARD variant | robust | robust | fragile | median | The S99/SC99 suffix indicates a flood-fragile assignment and likely signifies a nonstandard steel specification, meaning that flood performance cannot be assumed. |
| GROUP 7 — Wood (W+): 11 entries ¦ FLOOD ALWAYS FRAGILE; earthquake/wind varies by subtype |  |  |  |  |  |  |  |
| 17 | W | Wood, generic (PAGER W) | median | median | fragile | fragile | Generic wood construction is median for earthquakes (it is lightweight and flexible with moderate ductility). It is fragile in the event of a flood because wood swells and rots, which undermines the foundations, and in the event of a windstorm because the connections are inadequate and do not meet code. |
| 18 | W+WWD | Wood + wattle and daub | fragile | fragile | fragile | fragile | Wattle and daub is a construction technique involving thin woven branches that are plastered with mud or clay. It has near-zero lateral resistance. It is fragile and susceptible to all hazards. |
| GROUP 8 — Non-Steel Metal (ME): 3 entries ¦ All fragile |  |  |  |  |  |  |  |
| 19 | ME | Metal, except steel (generic) | fragile | fragile | fragile | fragile | Non-steel metals, such as iron and galvanized sheeting, are lightweight but lack the ductility of engineered steel. It is commonly used in informal settlements for roofing and walls without a structural frame. It is fragile to all hazards. |
| GROUP 9 — Masonry Unknown (M+): 3 entries |  |  |  |  |  |  |  |
| 20 | M+ADO | Masonry (unknown reinforcement) + adobe | fragile | fragile | fragile | fragile | The adobe block dominates performance regardless of the reinforcement status. |
| GROUP 10 — Mixed Construction (MIX, W+S): 15 entries ¦ Varies by component quality |  |  |  |  |  |  |  |
| 21 | MIX(MUR-CR) | Mix: unreinforced masonry + reinforced concrete (Switzerland/Germany) | median | median | median | fragile | This type is specific for Switzerland and Germany, and a well-engineered hybrid. The RC component elevates the combined performance to the median level. The URM panel exposure makes it fragile to wind. |
| GROUP 11 — Residual Categories (MATO, INF, UNK, duplicates): 6 entries |  |  |  |  |  |  |  |
| 22 | UNK | Material unknown / not specified | median | median | fragile | median | Unknown material: median combined (conservative but not worst-case). |
| 23 | INF | Informal construction (Fiji, Solomon Islands, Vanuatu) | fragile | fragile | fragile | fragile | It is explicitly informal and occurs only in Pacific island nations. It is fragile across all hazards. |

# Appendix E. List of economies dropped from the analysis due to lack of data

Several economies were excluded from the analysis due to missing data on key indicators, such as the poverty gap, construction quality index, GDP per capita, or hazard exposure measures. These exclusions encompass a mix of small island states, high-income jurisdictions, and economies with limited data infrastructure. Nevertheless, the final sample of 150 economies—each with a complete set of variables—remains geographically and economically diverse, supporting the validity of global insights and generalizable findings. From the World Bank country list, the following economies were excluded from the sample, with the specific missing variables indicated for each:

Table A-10: List of economies excluded from the sample

| Economy | GEM | Poverty | GDP | WGI | BQCI | Disaster |
| --- | --- | --- | --- | --- | --- | --- |
| Afghanistan | OK | Missing | OK | OK | OK | OK |
| American Samoa | OK | Missing | Missing | OK | Missing | OK |
| Andorra | OK | Missing | OK | OK | Missing | Missing |
| Antigua and Barbuda | OK | Missing | OK | OK | OK | OK |
| Aruba | OK | Missing | OK | OK | Missing | Missing |
| Azerbaijan | OK | Missing | OK | OK | OK | OK |
| Bahamas, The | OK | Missing | OK | OK | OK | OK |
| Bahrain | OK | Missing | OK | OK | OK | Missing |
| Bermuda | Missing | Missing | OK | OK | Missing | OK |
| British Virgin Islands | OK | Missing | Missing | Missing | Missing | OK |
| Brunei Darussalam | OK | Missing | OK | OK | OK | OK |
| Cabo Verde | OK | OK | OK | Missing | OK | OK |
| Cambodia | OK | Missing | OK | OK | OK | OK |
| Cayman Islands | OK | Missing | OK | OK | Missing | OK |
| Channel Islands | Missing | Missing | Missing | Missing | Missing | Missing |
| Côte d'Ivoire | OK | OK | OK | Missing | Missing | Missing |
| Cuba | OK | Missing | Missing | OK | Missing | OK |
| Curacao | Missing | Missing | OK | Missing | Missing | Missing |
| Czechia | OK | OK | OK | Missing | Missing | OK |
| Dominica | OK | Missing | OK | OK | OK | OK |
| Equatorial Guinea | OK | OK | OK | OK | OK | Missing |
| Eritrea | OK | Missing | Missing | OK | Missing | OK |
| Faroe Islands | Missing | Missing | OK | Missing | Missing | Missing |
| French Polynesia | Missing | Missing | Missing | Missing | Missing | OK |
| Gibraltar | OK | Missing | Missing | Missing | Missing | Missing |
| Greenland | Missing | Missing | OK | OK | Missing | Missing |
| Guam | OK | Missing | Missing | OK | Missing | OK |
| Guyana | OK | Missing | OK | OK | OK | OK |
| Hong Kong SAR, China | OK | Missing | OK | OK | OK | OK |
| Isle of Man | OK | Missing | Missing | Missing | Missing | Missing |
| Korea, Dem. People's Rep. | OK | Missing | Missing | Missing | Missing | OK |
| Kosovo | OK | OK | OK | OK | OK | Missing |
| Kuwait | OK | Missing | OK | OK | OK | OK |
| Libya | OK | Missing | OK | OK | Missing | OK |
| Liechtenstein | OK | Missing | Missing | OK | Missing | Missing |
| Macao SAR, China | OK | Missing | OK | OK | Missing | OK |
| Maldives | Missing | OK | OK | OK | OK | OK |
| Malta | OK | OK | OK | OK | OK | Missing |
| Monaco | OK | Missing | Missing | OK | Missing | Missing |
| Nauru | OK | OK | OK | OK | Missing | Missing |
| New Caledonia | OK | Missing | Missing | Missing | Missing | OK |
| New Zealand | OK | Missing | OK | OK | OK | OK |
| Northern Mariana Islands | OK | Missing | Missing | Missing | Missing | OK |
| Oman | OK | Missing | OK | OK | OK | OK |
| Palau | OK | Missing | OK | OK | OK | OK |
| Papua New Guinea | OK | Missing | OK | OK | OK | OK |
| Puerto Rico (US) | OK | Missing | OK | Missing | Missing | OK |
| San Marino | Missing | Missing | OK | OK | OK | Missing |
| São Tomé and Príncipe | OK | OK | OK | Missing | Missing | Missing |
| Saudi Arabia | OK | Missing | OK | OK | OK | OK |
| Singapore | OK | Missing | OK | OK | OK | Missing |
| Sint Maarten (Dutch part) | Missing | Missing | OK | Missing | Missing | OK |
| Somalia, Fed. Rep. | OK | Missing | OK | Missing | Missing | OK |
| South Sudan | OK | OK | Missing | OK | OK | OK |
| St. Kitts and Nevis | OK | Missing | OK | OK | OK | OK |
| St. Martin (French part) | Missing | Missing | Missing | Missing | Missing | OK |
| St. Vincent and the Grenadines | OK | Missing | OK | OK | OK | OK |
| Syrian Arab Republic | OK | OK | OK | OK | Missing | OK |
| Trinidad and Tobago | OK | Missing | OK | OK | OK | OK |
| Türkiye | OK | OK | OK | Missing | Missing | OK |
| Turkmenistan | OK | Missing | OK | OK | Missing | OK |
| Turks and Caicos Islands | OK | Missing | OK | Missing | Missing | OK |
| Tuvalu | OK | OK | OK | OK | Missing | OK |
| Venezuela, RB | OK | Missing | Missing | OK | OK | OK |
| Viet Nam | OK | OK | OK | OK | Missing | OK |
| Virgin Islands (U.S.) | OK | Missing | OK | OK | Missing | OK |
| Yemen, Rep. | OK | OK | Missing | OK | Missing | OK |

The number shows a share of non-fragile (robust + medium) housing in each country calculated by the author using the Global Exposure Model from GEM Foundation as discussed in Section 3.1.